| Product name | Confidentiality level |
| --- | --- |
| B593s-22 | CONFIDENTIAL |
| Product version | Total 9 pages |
| V1.2 |  |

B593s-22TCPU-V200R001B270D25SP01C260

Release Notes

V1.2

| Prepared by | Guo xiaofei | Date | 2016-12-13 |
| --- | --- | --- | --- |
| Reviewed by | Li jinze | Date | 2016-12-13 |
| Approved by | Panlianggang | Date | 2016-12-13 |

Huawei Technologies Co., Ltd.

All rights reserved

Revision Record

| Date | Revision version | FW-WebUI/HiLink Version | Change Description | Author |
| --- | --- | --- | --- | --- |
| 2013-7-16 | V1.0 | V200R001B180D20SP05C260 | The 1st version | xujun |
| 2015-10-15 | V1.1 | V200R001B270D25SP00C260 | The 2nd version | Guo xiaofei |
| 2016-12-13 | V1.2 | V200R001B270D25SP01C260 | The 3rd version | Guo xiaofei |

Table of Contents

1	Main Features	4

2	Hardware	4

2.1	Version Description	4

2.2	Hardware Specifications	4

2.3	Improvements in the Previous Version	6

2.4	Known Limitations and Issues	6

3	Firmware	6

3.1	Version Description	6

3.2	Firmware Specifications	6

3.3	Improvement in the Previous Version	8

3.4	Known Limitations and Issues	9

4	Software Vulnerabilities Fixes	9

B593s-22TCPU-V200R001B270D25SP01C260 Release Notes V1.2

# Main Features

- The B593s-22 supports the following standards:

- Support network environments. The B593s-22 supports LTE FDD Band 1(2100MHz) /Band 3(1800MHz) /Band 7(2600MHz) /Band 8(900MHz)/Band 20(800MHz)，LTE TDD Band38(2600MHZ)，UMTS FDD Band8(900 MHz)/Band 1(2100MHz)，GSM 1900/1800/900/850 MHz .

- High speed experience. Supports maximum throughput of 135 Mbps in the downlink and 50 Mbps in the uplink.

- Built-in DHCP Server and NAT. Provides high-speed routing capability.

- Comprehensive and robust security services. Provides instant protection to block potential security risks and intrusion attempts.

- Supports Windows XP, Windows Vista, Windows 7, Linux, MAC. Compatible with Browser include Internet Explorer, Firefox, Chrome, Safari, Opera.

- Intuitionist and convenient Web-based management.

- Ultrathin and artful appearance.

- Built-in LTE and WLAN high gain antenna. Ensured performance and easy portability. It is optional to select an external antenna, which is to ensure the normal use when the signal strength is weak.

- User-friendly design of LED indicator. Easy to observe the status of equipment.

# Hardware

## Version Description

| Hardware Version: | Ver.B |
| --- | --- |
| Platform & Chipset: | Hi6920 |

## Hardware Specifications

| Item | Specifications |  |  |
| --- | --- | --- | --- |
| Technical Standard | WAN: LTE/UMTS |  |  |
|  | LAN: IEEE 802.3/802.3u |  |  |
|  | WLAN: IEEE 802.11b/g/n |  |  |
| Operating Frequency | - LTE: FDD 800/900/1800/2100/2600 MHz, TDD 2600 MHz |  |  |
|  | - UMTS: 900/2100 MHz |  |  |
|  | - WLAN: 2.401–2.483 GHz |  |  |
| Maximum Transmitter Power | LTE |  | FDD 800 MHz: 22.6 dBm |
|  |  |  | FDD 900 MHz: 22.6 dBm |
|  |  |  | FDD 1800 MHz: 22.5 dBm |
|  |  |  | FDD 2100 MHz: 22.4 dBm |
|  |  |  | FDD 2600 MHz: 22.3 dBm |
|  |  |  | TDD 2600 MHz: 22.3 dBm |
|  | UMTS |  | 900 MHz: 23.6dBm |
|  |  |  | 2100 MHz: 23.5dBm |
|  | GSM |  | GSM 850: Power Class 4 31.6dBm |
|  |  |  | GSM 900: Power Class 4 31.6dBm |
|  |  |  | DCS 1800:Power Class 1 29.5dBm |
|  |  |  | PCS 1900:Power Class 1 29.5dBm |
|  | WLAN |  | - 802.11n: 13 dBm |
|  |  |  | - 802.11g: 16 dBm |
|  |  |  | - 802.11b: 16 dBm |
| Maximum Power Consumption | ≤ 11 W (adapter powered) |  |  |
| Memory | Flash 512 MB, DDR 256 MB |  |  |
| External Interfaces | - 4 Ethernet interface |  |  |
|  | - 2 POTS interface |  |  |
|  | - 1 Power interface |  |  |
|  | - 1 USIM card interface |  |  |
|  | - 2 External antenna interface |  |  |
|  | - 1 USB interface |  |  |
|  | - 1 Reset button |  |  |
|  | - 1 WLAN button |  |  |
|  | - 1 WPS button |  |  |
|  | - 1 Power button |  |  |
| Antenna | - Built-in LTE/UMTS antenna |  |  |
|  | - Built-out LTE/UMTS antenna (optional) |  |  |
|  | - Built-in WLAN antenna |  |  |
| Static Receiver Sensitivity | - LTE | - FDD 800 MHz: -94 dBm@10 MHz |  |
|  |  | - FDD 900 MHz: -94 dBm@10 MHz |  |
|  |  | - FDD 1800 MHz: -94 dBm@10 MHz |  |
|  |  | - FDD 2100 MHz: -97 dBm@10 MHz |  |
|  |  | - FDD 2600 MHz: -95 dBm@10 MHz |  |
|  |  | - TDD 2600 MHz: -97 dBm@10 MHz |  |
|  | - UMTS | - 900 MHz: ≤ -108.5 dBm@3.84 MHz |  |
|  |  | - 2100 MHz: ≤ -108.4 dBm@3.84 MHz |  |
|  | - GSM | - GSM850: ≤-107.6dBm/200KHz |  |
|  |  | - GSM900: ≤-107.6dBm/200KHz |  |
|  |  | - GSM1800: ≤-107.5dBm/200KHz |  |
|  |  | - GSM1900: ≤-107.5dBm/200KHz |  |
|  | - WLAN | - 802.11n: -64 dBm@65 Mbit/s |  |
|  |  | - 802.11g: -65 dBm@54 Mbit/s |  |
|  |  | - 802.11b: -76 dBm@11 Mbit/s |  |
| Power Supply | - AC: 100–240 V, 50/60 Hz |  |  |
|  | - DC: 12 V, 2A |  |  |
| Dimensions (H × W × D) | 176 mm × 190 mm × 35 mm |  |  |
| Weight | About 390 g(power adapter excluded) |  |  |
| Ambient Temperature | - Working temperature: 0°C to +40°C - Storage temperature: -20°C to +70°C |  |  |
| Humidity | 10% to 95% |  |  |

## Improvements in the Previous Version

| Index | Case ID | Issue Description |
| --- | --- | --- |
| NA |  |  |

## Known Limitations and Issues

| Index | Case ID | Issue Description |
| --- | --- | --- |
| NA |  |  |

# Firmware

## Version Description

| Firmware Version: | V200R001B270D25SP01C260 |
| --- | --- |
| Baseline information | Hisilicon B270C35. |
| OS | Linux |

## Firmware Specifications

| Item | Description |
| --- | --- |
| Gateway | Router: Support the default routing (the routing address is 0.0.0.0). |
|  | Support ARP |
|  | Support DNS |
|  | Support DDNS |
|  | Support ICMP |
|  | NAT: - Support NAT, NAPT (compliant with RFC2663, RFC3022 and RFC3027) - Support fragment message identification for normal NAT - Support NAT traverse of SIP |
|  | DHCP Server: - The default IP addresses of the DHCP server is from 192.168.1.1 to 192.168.1.254. The default gateway address is 192.168.1.1 - The default DHCP lease time is 24 hours - The DHCP Server can be enabled or disabled - The address pool of the DHCP server can be configured - The lease time can be configured - The IP address status can be displayed, such as the host name, MAC address, IP address, and remaining lease - Support static IP reserve |
| USB Storage | - Support FTP |
|  | - Support Samba |
|  | - Support DLNA |
| Data service | - Downlink FDD LTE packet data service of up to 135 Mbit/s - Uplink FDD LTE packet data service of up to 50 Mbit/s - Downlink TDD LTE packet data service of up to 80 Mbit/s (configuration 1) - Uplink TDD LTE packet data service of up to 20 Mbit/s (configuration 1) - Downlink TDD LTE packet data service of up to 112 Mbit/s (configuration 2) - Uplink TDD LTE packet data service of up to 10 Mbit/s (configuration 2) - DC-HSPA+ downlink packet data service at a maximum transmission rate of 43.2 Mbit/s - HSPA+ downlink packet data service at a maximum transmission rate of 21.6 Mbit/s - HSPA downlink packet data service at a maximum transmission rate of 14.4 Mbit/s - DC-HSPA+/HSPA+/HSPA uplink packet data service at a maximum transmission rate of 5.76 Mbit/s |
| LAN | 10 Mbit/s and 100 Mbit/s auto-negotiation |
|  | MDI/MDIX auto-sensing |
|  | IEEE802.3/802.3u is compatible |
| WLAN | Broadcast and hide the SSID |
|  | Authentication: - Open System and shared key authentication - 64/128-digit WEP encryption - 256-digit WPA-PSK/ WPA2-PSK encryption - TKIP ciphering algorithm - AES ciphering algorithm - TKIP and AES ciphering algorithm synchronously |
|  | MAC address authentication: up to 8 MAC address items |
|  | Ratio adjustment: - Automatically |
|  | STA management: support limit of access users (up to 32 users) |
| Access Device Management | Manage access device |
| Upgrade | Auto http upgrade and local upgrade |
| SIM | PIN Management, SIM card Authentication |
| Dial-up | Support automatic, manual connect |
| Configuration import and export | Encryption backup current settings and restore the backup settings |
| SNTP | Support DST(Daylight Saving Time) |
| Maintenance | Export current diagnosis results and operation logs |
| System requirement | CPU: Pentium 500 MHz or above Memory: 128 MB RAM or above Hard disk: 50 MB free disk space OS: Windows XP/Windows Vista/ Windows 7/ Linux/Mac Display resolution: 800 × 600 or above (1024 × 768 is recommended) Internet Explorer: IE7.0,8.0/firefox3.6/safari5.0/opera10/chrome8 or above |

## Improvement in the Previous Version

| Index | Features Description |
| --- | --- |
| 1 | New requirement of network mode |
| 2 |  |

## Known Limitations and Issues

| Index | Case ID | Issue Description |
| --- | --- | --- |
| 1 | NA |  |

# Software Vulnerabilities Fixes

| Software/Module name | Version | CVE ID | Vulnerability Description | Solution |
| --- | --- | --- | --- | --- |
| NA |  |  |  | Baseline already has finished developing with security issues, not relate to this one. |