| Product name | Confidentiality level |
| --- | --- |
| E303Fi-1 | CONFIDENTIAL |
| Product version | Total 6 pages |
| V200R002 |  |

E303Fi-1TCPU-21.521.29.00.00

Release Notes

| Prepared by | E303Fi-1Team |  | Date | 2015-1-14 |
| --- | --- | --- | --- | --- |
| Reviewed by | E303Fi-1Team |  | Date | 2015-1-14 |

Huawei Technologies Co., Ltd.

All rights reserved

Revision History

| Date | Revision Version | Change Description | Author |
| --- | --- | --- | --- |
| 2015-1-14 | 1.00 | Initial | E303Fi-1team |

Table of Contents

1	Main Features	4

2	Hardware	4

2.1	Version Description: CU1E3131IM	4

2.2	Hardware specifications	4

2.3	Improvements in the previous version	5

2.4	Known limitations and issues	5

3	Firmware	5

3.1	Version Description: 21.521.29.00.00	5

3.2	Software specifications	5

3.3	New features	6

3.4	Improvements in the previous version	6

3.5	Known limitations and issues	6

E303Fi-1TCPU-21.521.29.00.00 Release Notes

# Main Features

- The E303Fi-1 supports the following standards:

- HSDPA packet data service of up to 14.4 Mbit/s

- HSUPA packet data service of up to 5.76 Mbit/s

- WCDMA PS domain data service of up to 384 Kbit/s

- EDGE packet data service of up to 236.8 Kbit/s

- GPRS packet data service of up to 85.6 Kbit/s

- SMS based on CS/Packet Switched (PS) domain of GSM and WCDMA

- Plug and play (PnP)

- USSD Service

- Micro Secure Digital Memory Card up to 32GB

- Standard USB interface, auto installation

# Hardware

## Version Description: CU1E3131IM

## Hardware specifications

| Item | Specifications |
| --- | --- |
| Technical standard | - WCDMA /HSPA/HSDPA - HSUPA - GSM/GPRS/EGPRS |
| External interfaces | - USB interface: USB 2.0 high-speed interface |
|  | LED: indicating the status of the network |
|  | SIM/USIM card: standard 6-pin SIM card interface |
|  | SD card interface: Standard micro SD card interface |
| Maximum transmitter power | HSPA/HSDPA/HSUPA/ WCDMA: +22 dBm (Power Class 3) |
|  | GSM/GPRS 850/900 MHz: +32 dBm (Power Class 4) |
|  | GSM/GPRS 1800 MHz/1900 MHz: +29 dBm (Power Class 1) |
|  | EDGE 850/900MHz: +26 dBm (Power Class E2) |
|  | EDGE 1800MHz/1900MHz: +25 dBm (Power Class E2) |
| Static receiver sensitivity | HSPA+/HSDPA/HSUPA/WCDMA compliant with 3GPP TS 25.101 (R7) |
|  | GSM/GPRS/EDGE 850/900/1800/1900 MHz: compliant with 3GPP TS 05.05 (R99) |
| Dimensions (D × W × H) | - 85.5X27.08X12.11mm |
| Weight | < 30 g |
| Note: 3GPP = The 3rd Generation Partnership Project LED = light-emitting diode SIM = subscriber identity module TS = technical specification USIM = UMTS subscriber identity module |  |

## Improvements in the previous version

None.

## Known limitations and issues

None.

# Firmware

## Version Description: 21.521.29.00.00

## Software specifications

| Item | Specifications |
| --- | --- |
| Data service | GPRS: - Uplink: 85.6 kbit/s - Downlink: 85.6 kbit/s EDGE: - Uplink: 236.8 kbit/s - Downlink: 236.8 kbit/s WCDMA PS domain: - Uplink: 384 kbit/s - Downlink: 384 kbit/s - HSDPA: - 14.4 Mbit/s HSUPA: - 5.76 Mbit/s |
| SMS | - Based on CS domain/PS domain of GSM and WCDMA |

## New features

| No. | Description |
| --- | --- |

## Improvements in the previous version

| Version | Description |
| --- | --- |
| 21.521.29.00.00 | - First release version |

## Known limitations and issues

| No. | Description |
| --- | --- |