| Product name | Confidentiality level |
| --- | --- |
| E3131 | CONFIDENTIAL |
| Product version | Total 7 pages |
| V200R001 |  |

E3131i-81TCPU-21.521.31.00.00    Release Notes

| Prepared by | Liuchengwei | Date | 2015-1-22 |
| --- | --- | --- | --- |
| Reviewed by |  | Date |  |
| Approved by |  | Date |  |

Huawei Technologies Co., Ltd.

All rights reserved

Revision Record

| Date | Revision version | FW-WebUI/HiLink Version | Change Description | Author |
| --- | --- | --- | --- | --- |
| 2015-1-22 | 1.0 | FW V200R001B521D31SP00C00 | Build 1.1 Version | Liuchengwei |

Table of Contents

1	Main Features	4

2	Hardware	4

2.1	Version Description	4

2.2	Hardware Specifications	5

2.3	Improvements in the Previous Version	6

2.4	Known Limitations and Issues	6

3	Firmware	6

3.1	Version Description	6

3.2	Firmware Specifications	7

3.3	Improvement in the Previous Version	7

3.4	Known Limitations and Issues	7

E3131i-81TCPU-21.521.31.00.00 Release Notes

| - Abbreviations | description |
| --- | --- |
| 3GPP | The 3rd Generation Partnership Project |
| EDGE | - Enhanced GPRS |
| - LED | - Light-emitting diode |
| - SIM | - Subscriber identity module |
| - TS | - Technical specification |
| - USIM | - UMTS subscriber identity module |

# Main Features

- The E3131i-81( W2100/W1900/W900) supports the following standards:

- HSPA+/HSDPA/HSUPA/WCDMA 2100/1900/900/

- and GSM/GPRS/EDGE 850/900/1800/1900 MHz

- HSPA+/HSDPA packet data service of up to 21.6 Mbit/s

- HSUPA packet data service of up to 5.76 Mbit/s

- WCDMA PS domain data service of up to 384 Kbit/s

- EDGE packet data service of up to 236.8 Kbit/s

- GPRS packet data service of up to 85.6 Kbit/s

- SMS based on CS/Packet Switched (PS) domain of GSM and WCDMA, supporting          group sending of messages

- Plug and play (PnP)

- Micro Secure Digital Memory Card up to 32GB

- Standard USB interface, auto installation

- USB Stick, easy to connect

- HiLink features (Driverless, Zero installation, Auto connect)

# Hardware

## Version Description

| Hardware Version: CU1E3131IM.VER A |  |
| --- | --- |
| Platform & Chipset: Balong V3R3M BB : Hi6758M RFIC: Hi6361，Hi6361s |  |

## Hardware Specifications

| Item | Specifications |
| --- | --- |
| Technical standard | - HSPA+/HSDPA/ WCDMA R7 - HSUPA R6 - GSM/GPRS/EGPRS R99 |
| Operating frequency | HSPA+/HSDPA/HSUPA/ WCDMA 2100 MHz: - Uplink: 1920–1980 MHz - Downlink: 2110–2170 MHz HSPA+/HSDPA/HSUPA/ WCDMA 1900 MHz: - Uplink: 1850–1910 MHz - Downlink: 1930–1990 MHz - HSPA+/HSDPA/HSUPA/ WCDMA 900 MHz: - Uplink: 880–915 MHz - Downlink: 925–960 MHz GSM/GPRS/EDGE 850 MHz: - Uplink: 824–849 MHz - Downlink: 869–894 MHz GSM/GPRS/EDGE 900 MHz: - Uplink: 880–915 MHz - Downlink: 925–960 MHz GSM/GPRS/EDGE 1800 MHz: - Uplink: 1710–1785 MHz - Downlink: 1805–1880 MHz GSM/GPRS/EDGE 1900 MHz: - Uplink: 1850–1910 MHz - Downlink: 1930–1990 MHz |
| External interfaces | - USB interface: USB 2.0 high-speed interface |
|  | LED: indicating the status of the network |
|  | SIM/USIM card: standard 6-pin SIM card interface |
| Maximum transmitter power | HSPA+/HSDPA/HSUPA/ WCDMA 2100/1900/900MHz: +22 dBm (Power Class 3) |
|  | GSM/GPRS 850/900 MHz: +33 dBm (Power Class 4) |
|  | GSM/GPRS 1800 MHz/1900 MHz: +30 dBm (Power Class 1) |
|  | EDGE 850/900MHz: +27 dBm (Power Class E2) |
|  | EDGE 1800MHz/1900MHz: +26 dBm (Power Class E2) |
| Static receiver sensitivity | HSPA+/HSDPA/HSUPA/WCDMA 2100/1900/900MHz: compliant with 3GPP TS 25.101 (R7) |
|  | GSM/GPRS/EDGE 850/900/1800/1900 MHz: compliant with 3GPP TS 05.05 (R99) |
| Maximum power consumption | 2.5W |
| Power supply | 5 V/500 mA (USB interfaces) |
| Dimensions (D × W × H) | 85.4X27X12.5mm |
| Weight | <30g |
| Note: 3GPP = The 3rd Generation Partnership Project EDGE = enhanced GPRS LED = light-emitting diode SIM = subscriber identity module TS = technical specification USIM = UMTS subscriber identity module |  |

## Improvements in the Previous Version

| Index | Case ID | Issue Description |
| --- | --- | --- |
| NA |  |  |

## Known Limitations and Issues

| Index | Case ID | Issue Description |
| --- | --- | --- |
| NA |  |  |

# Firmware

## Version Description

| Firmware Version: | 21.521.31.00.00 |
| --- | --- |
| Baseline information : | Balong V3R3C50B210 |
| Windows Driver Version : | 5.05.02.00 |
| MAC Driver Version : Linux Driver Version : USB PID/VID: Switch Before : MBIM : 12D1/0x15CD No MBIM: 12D1/0x15CA, Switch After: 12D1/1506 Upgrade tools: 31.116.0.0 Autorun: 2.0.0.62 | 5.01.09.00 4.19.22.00 |

## Firmware Specifications

| Item | Specifications |
| --- | --- |
| 1 | - 支持安全升级 |

## Improvement in the Previous Version

| Index | Case ID | Issue Description |
| --- | --- | --- |
| NA |  |  |

## Known Limitations and Issues

| Index | Case ID | Issue Description |
| --- | --- | --- |
| NA |  |  |