| Product name | Confidentiality level |
| --- | --- |
| E8372h | CONFIDENTIAL |
| Product version | Total 6 pages |
| V1.0 |  |

E8372h Firmware Release Notes

V1.0

| Prepared by | Zonghaitao | Date | 2015-3-20 |
| --- | --- | --- | --- |
| Reviewed by | Xujun,dinglinhua | Date | 2015-3-20 |
| Approved by |  | Date |  |

Huawei Technologies Co., Ltd.

All rights reserved

Revision Record

| Date | Revision version | FW-WebUI/Stick Version | Change Description | Author |
| --- | --- | --- | --- | --- |
| 2015-2-16 | 1.0 | 21.200.05.00.00 | First version | zonghaitao |
| 2015-3-20 | 1.0 | 21.200.07.00.00 | Second version | zonghaitao |

Table of Contents

1	Main Features	4

2	Hardware	4

2.1	Hardware Specifications	4

3	Firmware	5

3.1	Version Description	5

3.2	Firmware Specifications	5

3.3	Improvement in the Previous Version	6

3.4	Known Limitations and Issues	6

E8372h Firmware Release Notes V1.0

# Main Features

- The E8372h supports the following standards:

- LTE cat4 data service up to 150Mbit/s (Downlink) and 50Mbit/s(Uplink)

- DC-HSPA+ data service up to 43.2 Mbit/s

- HSPA+ data service up to 21.6 Mbit/s

- HSDPA packet data service of up to 14.4 Mbit/s

- HSUPA data service up to 5.76 Mbit/s

- WCDMA PS domain data service of up to 384 Kbit/s

- Equalizer and receive diversity

- microSD Card Slot (Up to 32G)

- Data and SMS Service

- Plug and play

- Standard USB interface

- CSFB

# Hardware

## Hardware Specifications

| Item | Specifications |
| --- | --- |
| Hardware Version | - CL1E8372HM |
| Technical standard | - LTE 3GPP R9 - HSPA+/UMTS: 3GPP R99/R5/R6/R7/R8 - GSM/GPRS/EDGE: 3GPP R99 |
| External interfaces | - USB: Type A with standard USB 2.0 High speed interface |
|  | LED: indicating the status of the Data Card |
|  | SD card: standard TF card interface |
|  | SIM/USIM card: standard 6-pin SIM card interface |
|  | RF interface: external RF interface |
| Maximum power consumption |  3.5 W |
| Power supply | 5V |
| Dimensions (D × W × H) | - About 94mm(D) × 30mm(W) × 14mm (H) |
| Weight |  50g |
| Temperature | - Operating: –10℃ to +40℃ - Storage: –20℃ to +70℃ |
| Humidity | 5% to 95% |
| Base Information | - Plug and play (PnP) |
|  | - Standard USB 2.0 High Speed interface, auto installation, convenient for use |
| Note: 3GPP = The 3rd Generation Partnership Project TS = Technical Specification LED = Light-Emitting Diode SIM = Subscriber Identity Module USIM = UMTS Subscriber Identity Module |  |

# Firmware

## Version Description

| Firmware Version: | 21.200.07.00.00 |
| --- | --- |
| Baseline information | Balong V7R11 C30B300 |

## Firmware Specifications

| Firmware |  |  |
| --- | --- | --- |
| Item | Specifications |  |
| Version | 21.200.07.00.00 |  |
| Information | Platform | Balong V7R11 |
|  | Baseline | C30B300 |
|  | Feature | LTE Supported |
| Data service | GSM CS domain | - Uplink: 9.6/14.4 kbit/s - Downlink: 9.6/14.4 kbit/s |
|  | GPRS | - Uplink: 85.6 kbit/s - Downlink: 85.6 kbit/s |
|  | EDGE | - Uplink: 236.8 kbit/s - Downlink: 236.8 kbit/s |
|  | UMTS PS domain | - Uplink: 384 kbit/s - Downlink: 384 kbit/s |
|  | HSDPA | - 14.4Mbit/s |
|  | HSUPA | - 5.76 Mbit/s |
|  | HSPA+ | - 21.6Mbit/s |
|  | DC HSPA+ | - Uplink:5.76 Mbit/s - Downlink:43.2Mbit/s |
|  | LTE FDD | - Uplink: 50Mbit/s - Downlink:150Mbit/s |
| SMS | - SMS over SGs |  |
| Operating System | - Windows XP/Windows Vista/WIN 7/MAC/WIN8 |  |

## Improvement in the Previous Version

| Index | Case ID | Issue Description |
| --- | --- | --- |
| 1 |  |  |

## Known Limitations and Issues

| Index | Case ID | Issue Description |
| --- | --- | --- |