| Product name | Confidentiality level |
| --- | --- |
| E3372h | CONFIDENTIAL |
| Product version | Total 6 pages |
| V2.0 |  |

E3372h Firmware Release Notes

V2.0

| Prepared by | E3372h Team | Date | 2015-2-11 |
| --- | --- | --- | --- |
| Reviewed by | E3372h Team | Date | 2015-2-11 |

Huawei Technologies Co., Ltd.

All rights reserved

Revision Record

| Date | Revision version | change Description |  | Author |
| --- | --- | --- | --- | --- |
| 2015-1-8 | 1.0 | Release for Version 21.180.09.00.55 |  | E3372h Team |
| 2015-2-11 | 2.0 | Release for Version 21.180.09.01.55 |  | E3372h Team |

Table of Contents

1	Main Features	4

2	Hardware	4

2.1	Hardware Specifications	4

3	Firmware	5

3.1	Version Description	5

3.2	Firmware Specifications	5

3.3	Improvement in the Previous Version	6

3.3.1	Version 21.180.09.01.55	6

3.3.2	Version 21.180.09.00.55	6

3.4	Known Limitations and Issues	6

E3372h Firmware Release Notes V2.0

# Main Features

- The E3372h supports the following standards:

- LTE cat4 data service up to 150Mbit/s (Downlink) and 50Mbit/s(Uplink)

- DC-HSPA+ data service up to 43.2 Mbit/s

- HSPA+ data service up to 21.6 Mbit/s

- HSDPA packet data service of up to 14.4 Mbit/s

- HSUPA data service up to 5.76 Mbit/s

- WCDMA PS domain data service of up to 384 Kbit/s

- Equalizer and receive diversity

- microSD Card Slot (Up to 32G)

- Data and SMS Service

- Plug and play

- Standard USB interface

- CSFB

# Hardware

## Hardware Specifications

| Item | Specifications |
| --- | --- |
| Hardware Version | - CL2E3372HM |
| Technical standard | - LTE 3GPP R9 - HSPA+/UMTS: 3GPP R99/R5/R6/R7/R8 - GSM/GPRS/EDGE: 3GPP R99 |
| External interfaces | - USB: Type A with standard USB 2.0 High speed interface |
|  | LED: indicating the status of the Data Card |
|  | SD card: standard TF card interface |
|  | SIM/USIM card: standard 6-pin SIM card interface |
|  | RF interface: external RF interface |
| Maximum power consumption |  3.5 W |
| Power supply | 5V |
| Dimensions (D × W × H) | - About 88mm(D) × 28mm(W) × 11.5mm (H) |
| Weight |  25 g |
| Temperature | - Operating: –10℃ to +40℃ - Storage: –20℃ to +70℃ |
| Humidity | 5% to 95% |
| Base Information | - Plug and play (PnP) |
|  | - Standard USB 2.0 High Speed interface, auto installation, convenient for use |
| Note: 3GPP = The 3rd Generation Partnership Project TS = Technical Specification LED = Light-Emitting Diode SIM = Subscriber Identity Module USIM = UMTS Subscriber Identity Module |  |

# Firmware

## Version Description

| Firmware Version: | 21.180.09.01.55 |
| --- | --- |
| Baseline information | Balong V7R11 C30B180 |

## Firmware Specifications

| Firmware |  |  |
| --- | --- | --- |
| Item | Specifications |  |
| Version | 21.180.09.01.55 |  |
| Information | Platform | Balong V7R11 |
|  | Baseline | C30B180 |
|  | Feature | LTE Supported |
| Data service | GSM CS domain | - Uplink: 9.6/14.4 kbit/s - Downlink: 9.6/14.4 kbit/s |
|  | GPRS | - Uplink: 85.6 kbit/s - Downlink: 85.6 kbit/s |
|  | EDGE | - Uplink: 236.8 kbit/s - Downlink: 236.8 kbit/s |
|  | UMTS PS domain | - Uplink: 384 kbit/s - Downlink: 384 kbit/s |
|  | HSDPA | - 14.4Mbit/s |
|  | HSUPA | - 5.76 Mbit/s |
|  | HSPA+ | - 21.6Mbit/s |
|  | DC HSPA+ | - Uplink:5.76 Mbit/s - Downlink:43.2Mbit/s |
|  | LTE FDD | - Uplink: 50Mbit/s - Downlink:150Mbit/s |
| SMS | - SMS over SGs |  |
| Operating System | - Windows XP/Windows Vista/WIN 7/MAC/WIN8 |  |

## Improvement in the Previous Version

### Version 21.180.09.01.55

| Index | Case ID | Issue Description |
| --- | --- | --- |
| 1 | 60914 | - MNC 20 sims should be treated exactly the same way in the network as MNC 16 sims |

### Version 21.180.09.00.55

| Index | Case ID | Issue Description |
| --- | --- | --- |
| 1 | / | - Modify Operator Name as follows: - MCC+MNC Long name Short name - 234 30 ,31,32 "EE" "EE" - 262 01 "Telekom.de" "Telekom.de" - 230 01 "T-Mobile CZ" "T-Mobile CZ" - 232 03 "T-Mobile A" "T-Mobile A" - 232 05,10 "3 AT" "3 AT" - 232 12 "Yessss!" "Yessss!" - 232 07 "T-Mobile Telering" "tele.ring" - 204 16 "T-Mobile NL" "T-Mobile NL" - 204 20 "T-Mobile NL" "T-Mobile NL" - 219 01 "HT HR" "HT HR" - 216 30 "Telekom HU" "Telekom HU" - 231 02 "Telekom SK" "Telekom SK" - 294 01 "T-Mobile MK" "T-Mobile MK" - 297 02 "Telekom.me" "Telekom.me" - 260 02 "T-Mobile.pl" "T-Mobile.pl" - 226 03,06 "TELEKOM.RO" "TELEKOM.RO" - 202 01 "GR COSMOTE" "GR COSMOTE" - 284 05 "BG GLOBUL" "BG GLOBUL" - 276 01 "AMC - AL" "AMC - AL" |

## Known Limitations and Issues

NA