| Product name | Confidentiality level |
| --- | --- |
| E5573Cs-322 | CONFIDENTIAL |
| Product version | Total 8 pages |
| V1.1 |  |

HUAWEI E5573Cs-322TCPU-V200R001B326D62SP00C673

Release Notes V1.1

| Approved by | E5573Cs-322 Team | Date | 2017/05/31 |
| --- | --- | --- | --- |

Huawei Technologies Co., Ltd.

Revision Record

| Date | Revision version | FW-WebUI/HiLink Version | Change Description | Author |
| --- | --- | --- | --- | --- |
| 2017/05/31 | 1.0 | FW21.326.62.00.673 | The 1st Version | LMT |

Table of Contents

1	Main Features	4

2	Hardware	5

2.1	Version Description	5

2.2	Hardware Specifications	5

2.3	Improvements in the Previous Version	6

2.4	Known Limitations and Issues	6

3	Firmware	7

3.1	Version Description	7

3.2	Firmware Specifications	7

3.3	Improvement in the Previous Version	7

3.4	Known Limitations and Issues	7

4	WebUI/HiLink	7

4.1	Version Description	7

4.2	WebUI/HiLink Specifications	8

4.3	Improvement in the Previous Version	8

4.4	Known Limitations and Issues	8

5	Software Vulnerabilities Fixes	8

6	Accessory Product from other Vendor	8

7	Others	8

8	Reference	8

HUAWEI E5573Cs-322TCPU-V200R001B326D62SP00C673 Release Notes V1.1

| - Abbreviations | Description |
| --- | --- |

# Main Features

The E5573Cs-322 mainly supports the following features:

- LTE FDD (DL) data service of up to 100 Mbit/s

- LTE FDD (UL) data service of up to 50 Mbit/s

- DC-HSPA+ (DL) data service of up to 43.2 Mbit/s

- HSPA+ (DL) data service of up to 21.6 Mbit/s

- HSDPA (DL) data service of up to 14.4 Mbit/s

- HSUPA (UL) data service of up to 5.76 Mbit/s

- UMTS data service of up to 384 kbit/s

- EDGE data service of up to 236.8 kbit/s

- GPRS data service of up to 85.6 kbit/s

- PS domain data service based on LTE/UMTS/GSM

- SMS based on LTE/UMTS/GSM

- Built-in LTE/UMTS/GSM and WLAN high gain antenna

- Wi-Fi 2.4 GHz

- LTE/3G/Wi-Fi auto offload

- Support for HUAWEI HiLink App

- Press and Play

- IPv6/IPv4 dual stack (optional)

- Built-in DHCP Server, DNS RELAY and NAT

- Online software upgrade

- Traffic statistic

- Standard Micro USB interface

- 2D Barcode easy connection

- Windows 7, Windows 8, Windows 8.1 ,Windows 10 (does not support Windows RT), MAC OS X 10.7, 10.8, 10.9, 10.10 and 10.11 with latest upgrades

# Hardware

## Version Description

| Hardware Version: | CL1E5573CSM11 Ver.B |
| --- | --- |
| Platform & Chipset: | Balong Hi6921 V7R11M, Realtek 8189 |

## Hardware Specifications

| Item | Specifications |  |
| --- | --- | --- |
| Technical standard | - WAN: LTE FDD/DC-HSPA+/HSPA+/HSPA/UMTS/EDGE/GPRS/GSM |  |
|  | - WLAN: IEEE 802.11b/g/n |  |
| Operating frequency | LTE: FDD B1/B3/B7/B8/B20 |  |
|  | - DC-HSPA+/HSPA+/HSPA/UMTS: B1/B8 |  |
|  | - EDGE/GPRS/GSM: B2/B3/B5/B8 |  |
|  | - WLAN: 2.4 GHz |  |
| Internal memory | 128 MB Flash, 128 MB DDR SDRAM |  |
| Maximum transmitter power | LTE: Conform to Power Class 3 Definition |  |
|  | UMTS: Conform to Power Class 3 Definition |  |
|  | WLAN | 802.11b: 14 dBm |
|  |  | 802.11g: 11 dBm |
|  |  | 802.11n: 10 dBm |
| Receiver sensitivity | LTE: Confirm to 3GPP Requirements |  |
|  | UMTS: Confirm to 3GPP Requirements |  |
|  | WLAN | 802.11b: -76 dBm@11 Mbit/s |
|  |  | 802.11g: -65 dBm@54 Mbit/s |
|  |  | 802.11n: -64 dBm@65 Mbit/s |
| WLAN speed | 802.11b: Up to 11 Mbit/s |  |
|  | 802.11g: Up to 54 Mbit/s |  |
|  | 802.11n | HT20: Support MCS0–MCS7; Up to 72.2 Mbit/s. HT40: Support MCS0–MCS7; Up to 150 Mbit/s. |
| Maximum power consumption | 3.5 W |  |
| Power supply | - AC: 100–240 V |  |
|  | - DC: 5 V, 1 A |  |
| Battery | - Type: Li (rechargeable) |  |
|  | - Capacity: 3.8 V, 1500 mAh |  |
|  | - Maximum working time: 6 hours (depending on the network) |  |
|  | - Maximum standby time: 300 hours (depending on the network) |  |
| External interfaces | Micro USB interface |  |
|  | Standard 6-pin SIM card interface |  |
| Indicators | LED indicators: Signal, Battery |  |
| Key-press | Power switch, RESET switch |  |
| Antenna | - Built-in LTE/UMTS/GSM main antenna |  |
|  | - Built-in LTE/UMTS diversity antenna |  |
|  | - Built-in WLAN antenna |  |
| Dimensions (W × D × H) | 96.8 mm×58.0 mm×12.8 mm |  |
| Weight | about 75 g (including the battery) |  |
| Temperature | - Operating: 0℃ to +35℃ |  |
|  | - Storage: -20℃ to +60℃ |  |
| Humidity | 5% to 95% (non-condensing) |  |

## Improvements in the Previous Version

| Index | Case ID | Issue Description |
| --- | --- | --- |
| Hardware Version |  | CL1E5573CSM11 Ver.B |
| Previous Hardware Version |  | NA |
| NA |  |  |

## Known Limitations and Issues

| Index | Case ID | Issue Description |
| --- | --- | --- |
| NA |  | NA |

# Firmware

## Version Description

| Firmware Version: | 21.326.62.00.673 |
| --- | --- |
| Baseline information | Hi6921 V7R11M |

## Firmware Specifications

| Item | Specifications |
| --- | --- |
| WEB log In | IP:192.168.8.8 Username:admin Password:admin |
| WiFi parameter | SSID：ALTEL4G-****,**** is the last four digits of MAC，caps WIFI KEY：8 random number (only include number) |
| SIMLOCK | 40107 and 40177, unlock times:10 |

## Improvement in the Previous Version

| Index | Case ID | Issue Description |
| --- | --- | --- |
| Firmware Version |  | 21.326.62.00.673 |
| Previous Firmware Version |  | NA |

## Known Limitations and Issues

| Index | Case ID | Issue Description |
| --- | --- | --- |
| 1 | Unrealized Features | NA |
| 2 |  |  |
| 3 |  |  |

# WebUI/HiLink

## Version Description

| WebUI/HiLink Version: | NA |
| --- | --- |

## WebUI/HiLink Specifications

| Item | Specifications |
| --- | --- |

## Improvement in the Previous Version

| Index | Case ID | Issue Description |
| --- | --- | --- |
| WebUI Version |  |  |
| Previous WebUI Version |  |  |
| 1 | New Features |  |
| 2 |  |  |
| 3 |  |  |

## Known Limitations and Issues

| Index | Case ID | Issue Description |
| --- | --- | --- |
| 1 | Unrealized Features |  |
| 2 |  |  |
| 3 |  |  |

# Software Vulnerabilities Fixes

# Accessory Product from other Vendor

# Others

# Reference