| Product name | Confidentiality level |
| --- | --- |
| E5573Bs-320 | CONFIDENTIAL |
| Product version | Total 7 pages |
| V1.0 |  |

HUAWEI E5573Bs-320TCPU-V200R001B316D03SP00C1404

Release Notes V1.0

| Prepared by | E5573Bs-320 Team | Date | 2016-4-22 |
| --- | --- | --- | --- |
| Reviewed by | E5573Bs-320 Team | Date | 2016-4-22 |
| Approved by | E5573Bs-320 Team | Date | 2016-4-22 |

Huawei Technologies Co., Ltd.

Revision Record

| Date | Revision version | FW-WebUI/HiLink Version | Change Description | Author |
| --- | --- | --- | --- | --- |
| 2016-4-22 | 1.0 | FW 21.316.03.00.1404 | The 1st Version | E5573Bs-320 Team |

Table of Contents

1	Main Features	4

2	Hardware	4

2.1	Version Description	4

2.2	Hardware Specifications	4

2.3	Improvements in the Previous Version	5

2.4	Known Limitations and Issues	5

3	Firmware	5

3.1	Version Description	5

3.2	Firmware Specifications	5

3.3	Improvement in the Previous Version	6

3.4	Known Limitations and Issues	6

4	Software Vulnerabilities Fixes	6

5	Accessory Product from other Vendor	7

5.1	Version Description	7

5.2	Known Limitations and Issues	7

6	Others	7

7	Reference	7

HUAWEI E5573Bs-320TCPU-V200R001B316D03SP00C1404 Release Notes V1.0

| - Abbreviations | Description |
| --- | --- |

# Main Features

- The E5573Bs-320 supports the following features:

- LTE cat4 data service up to 150Mbit/s (Downlink) and 50Mbit/s(Uplink)

- DC-HSPA+ data service up to 43.2 Mbit/s

- HSPA+ data service up to 21.6 Mbit/s

- HSDPA packet data service of up to 14.4 Mbit/s

- HSUPA data service up to 5.76 Mbit/s

- WCDMA PS domain data service of up to 384 Kbit/s

- Equalizer and receive diversity

- Data and SMS Service

- WEB UI, Auto connect

- Plug and play

- Standard USB2.0

- Support WiFi 2.4GHz/5GHz

# Hardware

## Version Description

| Hardware Version: | CL1E5573SM10 Ver.A |
| --- | --- |
| Platform & Chipset: | Balong Hi6921 V7R11M, Broadcomm 43241 |

## Hardware Specifications

| Item | Specifications |  |
| --- | --- | --- |
| Technical Standard | 3GPP | R99/R5/R6/R7/R8/R9 |
|  | IEEE | 802.11b/g/n |
| Operating Frequency | LTE | FDD B1,B3,B5,B7,B8,B20 |
|  | UMTS | B1, B5,B8 |
| Maximum Transmitter Power | LTE | +23dBm (Class 3) |
|  | UMTS | +24dBm (Class 3) |
| Maximum Power Consumption | 3.5W |  |
| Memory | 128M NAND Flash, 128M DDR |  |
| WLAN Rate | 802.11b: 11Mbit/s, 5.5Mbit/s, 2Mbit/s, 1Mbit/s 802.11g: 54Mbit/s, 48Mbit/s, 36Mbit/s, 24Mbit/s, 18Mbit/s, 12Mbit/s, 9Mbit/s, 6Mbit/s 802.11n: MCS0-MCS7(WiFi 1x1), MSC0-MCS15(WiFi 2x2) |  |
| External Interfaces | - USB: Standard USB2.0 |  |
|  | LED |  |
|  | SIM/USIM card: 6pin, 1.8/3V |  |
| Display | - LED |  |
| Keys | 1 Power, 1 Reset |  |
| Antenna | Internal |  |
| Static Receiver Sensitivity | Compliant with 3GPP TS 36.101(R9) for LTE, TS 25.101(R8) for UMTS. |  |
| Battery | 1500mAh |  |
| Dimensions (D × W × H) | 96.8*58*12.8 mm |  |
| Weight | <120g(include Battery) |  |
| Ambient Temperature | 0-35°C |  |
| Humidity | 5%-95% |  |

## Improvements in the Previous Version

| Index | Case ID | Issue Description |
| --- | --- | --- |
| Hardware Version |  | CL1E5573SM10 Ver.A |
| Previous Hardware Version |  | NA |
| NA |  |  |

## Known Limitations and Issues

| Index | Case ID | Issue Description |
| --- | --- | --- |
| NA |  | NA |

# Firmware

## Version Description

| Firmware Version: | 21.316.03.00.1404 |
| --- | --- |
| Baseline information | Hi6921 V7R11M |

## Firmware Specifications

| Item | Specifications |
| --- | --- |

## Improvement in the Previous Version

| Index | Case ID | Issue Description |
| --- | --- | --- |
| Firmware Version |  | 21.316.03.00.1404 |
| Previous Firmware Version |  | 21.316.03.00.00 |
| 1 | WEB UI Log In | IP:192.168.1.1 User name:admin Password:admin |
| 2 | WLAN parameters | SSID: Ucom-****,**** is the last four digits of MAC WIFI KEY：8 random numbers |
| 3 | SIMLOCK | 28310 |

## Known Limitations and Issues

| Index | Case ID | Issue Description |
| --- | --- | --- |
| 1 | Unrealized Features | NA |
| 2 |  |  |
| 3 |  |  |

# Software Vulnerabilities Fixes

| Software/Module name | Version | CVE ID | Vulnerability Description | Solution |
| --- | --- | --- | --- | --- |
| Linux kernel | Linux kernel 3.14.1 | CVE-2014-2851 | 【P711s】CVE-2014-2851，CVE-2013-1763，CVE-2014-4943 漏洞合入 | Linux kernel 整数溢出漏洞，Linux kernel 3.14.1及之前版本的net/ipv4/ping.c文件中的‘ping_init_sock’函数存在整数溢出漏洞。本地攻击者可借助特制的应用程序利用该漏洞造成拒绝服务（释放后重用和系统崩溃）或获取特权。 |
| Linux Kernel | Linux kernel 3.7.10 | CVE-2013-1763 | 【P711s】CVE-2014-2851，CVE-2013-1763，CVE-2014-4943 漏洞合入 | Linux kernel 数组索引漏洞， Linux kernel 3.7.10之前的版本中的net/core/sock_diag.c中的‘__sock_diag_rcv_msg’函数中存在数组索引漏洞。本地攻击者通过含有较大的family值的Netlink消息，利用该漏洞获得特权。 |
| Linux kernel | Linux kernel 3.15.6 | CVE-2014-4943 | 【P711s】CVE-2014-2851，CVE-2013-1763，CVE-2014-4943 漏洞合入 | Linux kernel 3.15.6之前的版本中的net/l2tp/l2tp_ppp.c中 PPPoL2TP功能允许本地用户通过利用一个L2TP插座和插座INET之间的数据结构的差异来获得特权。 |
| Ipv4 | NA | ANDROID-20770158 | 【Android补丁】ANDROID-20770158： ipv4: Missing sk_nulls_node_init() in ping_unhash(). | 在ping_unhash()中缺少sk_nulls_node_init（）。 |
| Linux kernel | Linux kernel 3.4 | CVE-2015-1805 | 【P711s】CVE-2015-1805 漏洞合入 | https://android-review.googlesource.com/#/c/208731/ |

# Accessory Product from other Vendor

## Version Description

| Accessory Product Version: |  |
| --- | --- |

## Known Limitations and Issues

# Others

# Reference