Operating Manual

Software Option: 9225 Upgrade Tool Operation Instruction

Software Version: H

ZTE CORPORATION

Contents

1	Applicative context	3

1.Hardware environment	3

2.Software environment	3

2	Operatonal	3

2.1 Start Download Software	3

2.2 Settings And Operation Mode Select	4

2.2.1 Setting	4

2.2.2 Action Mode	4

2.2.3 Download information record	5

2.2.4 Help	5

2.3 Connecting Phone	6

2.4 Close	6

2.5 Notes	6

# Applicative context

## 1.Hardware environment

- computer configuration: Dual Core Processor 4000+  2.11GHZ          dominant frequency  1.99GB memory

- mobile type: 203Z/GL09P ,MF98N ,MF910，MF95

- country or platform: 9225

- data line type: 5Pin

- USB transmission speed:2.0+1.1

- Softdog:NO

- Other: NO

## 2.Software environment

- operating system: Windows® XP, Windows® 7 system.

- driver version: UI

- license file:NO

- third party software:NO

- other:NO

# Operatonal

## 2.1 Start Download Software

- Start the computer.

- Double click the SalesDL_LTE_9225V1.00.07.exe.

- The main interface of the software is shown as follows.

Note: Before update, ensure that the software and drivers have been installed correctly according to the modem user manual or quick start guide.

Figure 1

## 2.2 Settings And Operation Mode Select

### 2.2.1 Setting

The left side of the lower half part of the software is items of buttons & widgets that related to the setting, shown as the follows:

Figure 2

- 【SW Directory】 is used for select the root directory of Download Directory to be upgraded;

(2) 【NV Directory】is used for choose NV Backup path;

### 2.2.2 Action Mode

There are three modes: Download, Backup NV and Restore NV.

Figure 4

【Download】: The program will launch download process. The download flow is: backup NV, download the software, reset the phone, restore the NV

【Backup NV】: The program will backup NV only.

【Restore NV】: The program will restore NV only. Please make sure the backed NV files exist before restore.

### 2.2.3 Download information record

The information of download will be shown or exported through this area.

【】: Export the details of download information including Tool name, Device’s IMEI, Version before download, Version after download, Start time, End time, Download result(Success or not), PC’s HardDisk. The type of the exported file is *.csv. If you installed the Office, you can check it by Excel. Or you can check it through TXT.

【】: Export the details of download information which is encrypted.

【All Counts:】: Show the total number of downloading like this, Success Times/Total Times.

【Now Counts:】: Show the current number of downloading like this, Success Times/Total Times. Note that, the “Now Counts” reflect the download times from the “Last Clear Time”. And the “All Counts” reflect the download times from the first time you use this tool.

【Last Clear Time:】: Show the time you clear the record of “Now Counts”.

【Current Time:】: Show the current time.

【】: Clear the record of “Now Counts”.

### 2.2.4 Help

1. You can check the information of tool from. Then you will see picture as follows,

2. You can open the Help document through.

## 2.3 Connecting Phone

- Connect the data card with the computer. If the data card need battery, please insert battery and power on the data card.

- Make sure the correct drivers have already installed in your PC.

- Run the tool installed before. If your data card connects correctly, this tool will detect your data card automatically. Of course it will take for a while, about 15seconds. Please wait patiently.

- Now you can choose the software version and press “UPDATE” button to update.

## 2.4 Close

- The software cannot be closed when the download process is running.

- You can close the tool by pressing the button  or click the menu.

## 2.5 Notes

(1) This software supports the download of software for the MDM9225 modem.

(2) Firstly, remove your SIM card and T-Flash card from the modem, backup the phone book, SMS, personal information, etc .

(3) The update may fail if your computer goes to sleep, standby or hibernates during the update.

(4) Do not try to close the update tool or pop out the data card during the download process. Or the data card would be damaged.

(5)When this software circulates each time, is quasi- to circulate an example with solid procedure, can't open procedure for many times.

(6) If you have any question, please login http://ensupport.zte.com.cn