Technical Document

Document Name:  SalesDL_DC_MF80_COS_RO_help

Version : B

- Total  6  Pages

|  | Drafted by: | Wang Fengfeng |  |
| --- | --- | --- | --- |
|  | Checked by: |  |  |
|  | Signed by: |  |  |
|  | - Standardized by: |  |  |
|  | Approved by: |  |  |

Contents

1．Objective	3

2 Getting Started	3

2.1 Install Download Software	3

2.2 Settings and Operation Mode Select	3

2.2.1 Setting	3

2.2.2 Download information record	4

2.2.3 Help	4

2.3 Connecting Phone	5

2.4 Close	5

2.5 Notes	5

# 1．Objective

The Download Tool is the software which be run in PC with Windows® XP, Windows® 7 and Windows® 8.

The purpose of tool is updating the software in MF80 modem.

# 2 Getting Started

## 2.1 Install Download Software

- Start the computer. Run SalesDL_DC_MF80_COS_ROV1.00.01.exe

- The main interface of the software is shown as follows.

Figure 1

## 2.2 Settings and Operation Mode Select

### 2.2.1 Setting

The setting area shows as follow,

Figure 2

- 【SW Directory】: is used for select the root directory of Download Directory to be upgraded;

- 【NV Directory】: is used for choose NV Backup path;

- 【Download】: The program will launch download process. The download flow is: backup NV,  download the software, reset the phone, restore the NV

- 【Backup NV】: The program will backup NV only.

- 【Restore NV】: The program will restore NV only. Please make sure the backed NV files exist before restore.

### 2.2.2 Download information record

The information of download will be shown or exported through this area.

【】: Export the details of download information including Tool name, Device’s IMEI, Version before download, Version after download, Start time, End time, Download result(Success or not), PC’s HardDisk. The type of the exported file is *.csv. If you installed the Office, you can check it by Excel. Or you can check it through TXT.

【】: Export the details of download information which is encrypted.

【All Counts:】: Show the total number of downloading like this, Success Times/Total Times.

【Now Counts:】: Show the current number of downloading like this, Success Times/Total Times. Note that, the “Now Counts” reflect the download times from the “Last Clear Time”. And the “All Counts” reflect the download times from the first time you use this tool.

【Last Clear Time:】: Show the time you clear the record of “Now Counts”.

【Current Time:】: Show the current time.

【】: Clear the record of “Now Counts”.

### 2.2.3 Help

1. You can check the information of tool form. Then you will see dialog as follows,

2. You can open the Help document through.

## 2.3 Connecting Modem

- Connect the modem with the computer. If the modem needs battery, please insert battery and power on the modem.

- Make sure the correct drivers have already installed in your PC.

- Run the tool installed before. If your modem connects correctly, this tool will detect your modem automatically. Of course it will take for a while, about 15seconds. Please wait patiently.

- Now you can choose the software version and click “START” button to update.

- When the download process finished, yu will see the message box as follows:

- After download process successfully, please remove the modem first and then click the “OK” button to begin the next download process.

## 2.4 Close

- The software cannot be closed when the download process is running.

- You can close the tool by pressing the button  or click the menu.

## 2.5 Notes

(1) The software supports the free-driver modem to download.

(2) Once the modem is detected by the software, the modem will be changed from work to the port mode automatically.

(3) When the download process succeed, the modem will be changed to the work mode automatically after the software shows: “Update success! Please remove the modem!”, then you have to remove the modem before you click “OK”.

(4) When the software is closing, if there is a modem cennecting with the PC, the software will change the modem from port mode to work mode automatically. So, if you want to remove the modem without a download process, please close the software first, and then remove the modem in about 10 seconds later.

(5) This software supports the download of software for the current MF80 modem.

(6) If you try to open the software when it is running , will pop-up the dialog.

(7) Firstly, remove your SIM card and T-Flash card from the modem, backup the phone book, SMS, personal information, etc .

(8) The update may fail if your computer goes to sleep, standby or hibernates during the update.

(9) Do not try to close the update tool or pop out the modem during the download process. Or the modem would be damaged.

(10)If you have any question, please login http://support.zte.com.cn