| Product name | Confidentiality level |
| --- | --- |
| B310s-22 | CONFIDENTIAL |
| Product version | Total 13 pages |
| V1.1 |  |

B310s-22TCPU-V200R001B311D03SP02C141 Release Notes

V1.1

| Prepared by | Guo xiaofei | Date | 2015-9-8 |
| --- | --- | --- | --- |
| Reviewed by | Cheng qing | Date | 2015-9-8 |
| Approved by | Pan lianggang | Date | 2015-9-8 |

Huawei Technologies Co., Ltd.

All rights reserved

Revision Record

| Date | Revision version | FW-WebUI/HiLink Version | Change Description | Author |
| --- | --- | --- | --- | --- |
| 2015-7-8 | 1.0 | FW 21.311.03.00.141 | The 1th Version | Guo xiaofei |
| 2015-09-08 | 1.1 | FW 21.311.03.02.141 | The 2th Version | Guo xiaofei |

Table of Contents

1	Main Features	4

2	Hardware	5

2.1	Version Description	5

2.2	Hardware Specifications	5

2.3	Improvements in the Previous Version	6

2.4	Known Limitations and Issues	7

3	Firmware	7

3.1	Version Description	7

3.2	Firmware Specifications	7

3.3	Improvement in the Previous Version	8

3.4	Known Limitations and Issues	9

4	WebUI	9

4.1	Version Description	9

4.2	WebUI/HiLink Specifications	9

4.3	Improvement in the Previous Version	9

4.4	Known Limitations and Issues	9

5	Software Vulnerabilities Fixes	9

B310s-22TCPU-V200R001B311D03SP02C141 Release Notes V1.1

| - Abbreviations | description |
| --- | --- |

# Main Features

The B310s-22 mainly supports the following features:

- LTE FDD (DL) data service of up to 150 Mbit/s

- LTE FDD (UL) data service of up to 50 Mbit/s

- LTE TDD (DL) data service of up to 112 Mbit/s

- LTE TDD (UL) data service of up to 10 Mbit/s

- DC-HSPA+ (DL) data service of up to 42 Mbit/s

- HSPA+ (DL) data service of up to 21.6 Mbit/s

- HSDPA (DL) data service of up to 14.4 Mbit/s

- HSUPA (UL) data service of up to 5.76 Mbit/s

- UMTS data service of up to 384 kbit/s

- EDGE data service of up to 236.8 kbit/s

- EDGE data service of download to 296 kbit/s

- GPRS data service of up to 85.6 kbit/s

- PS domain data service based on LTE/UMTS/GSM

- SMS based on CS/PS domain of GSM and UMTS, CS domain of LTE

- Wi-Fi

- Support for HUAWEI Mobile WiFi App

- Press and Play

- IPv6v4 /IPv4 dual stack

- Built-in DHCP Server, DNS RELAY and NAT

- Online software upgrade

- Traffic statistic

- LED indicators

- Built-in UMTS and WLAN high gain antenna LTE/GSM

- Windows XP SP3, Windows Vista SP1/SP2, Windows 7, Windows 8, Windows 8.1 (does not support Windows RT), MAC OS X 10.7, 10.8 and 10.9 with latest upgrades

# Hardware

## Version Description

| Hardware Version: | WL1B310FM03 (B310s-22) |
| --- | --- |
| Platform & Chipset: | Balong Hi6921 & AR 8035 |

## Hardware Specifications

| Item | Specifications |  |  |  |
| --- | --- | --- | --- | --- |
| Technical standard | - WAN: LTE/ DC-HSPA+/HSPA+/HSPA/UMTS/EDGE/GPRS/GSM |  |  |  |
|  | - WLAN: IEEE 802.11b/g/n |  |  |  |
| Operating frequency | LTE: B1/B3/B7/B8/B20/B38 |  |  |  |
|  | HSPA+/HSPA/UMTS: B1/B8 |  |  |  |
|  | EDGE/GPRS/GSM: 1900/1800/900/850 MHz |  |  |  |
|  | WLAN: 2.4 GHz |  |  |  |
| Internal memory | 512 MB Flash,256 MB Memory |  |  |  |
| Maximum transmitter power | UMTS: 24 (+1/-3) dBm |  |  |  |
|  | WLAN |  | 802.11b: 20 (+/-3) dBm |  |
|  |  |  | 802.11g: 17 (+/-3) dBm |  |
|  |  |  | 802.11n: 17 (+/-3) dBm |  |
| Receiver sensitivity | UMTS: Confirm to 3GPP Requirements |  |  |  |
|  | WLAN 802.11b |  |  | -76 dBm@11 Mbit/s |
|  |  |  |  | -82 dBm@1 Mbit/s |
|  | WLAN 802.11g: -65 dBm@54 Mbit/s |  |  |  |
|  | WLAN 802.11n: -64 dBm@65 Mbit/s |  |  |  |
| WLAN speed | 802.11b: Up to 11 Mbit/s |  |  |  |
|  | 802.11g: Up to 54 Mbit/s |  |  |  |
|  | 802.11n: HT40 MCS15(300Mbit/s), HT20 MCS15(144.4Mbit/s) |  |  |  |
| Maximum power consumption | 12 W |  |  |  |
| Power supply | - AC: 100–240 V |  |  |  |
|  | - DC: 12 V, 1 A |  |  |  |
| External interfaces | WAN/LAN: 1 RJ45,GE FXS:1 RJ11 |  |  |  |
|  | SIM card interface: standard 6-pin SIM card interface |  |  |  |
| Indicators | Mode: | cyan: 4G mode blue: 3G mode yellow: 2G mode green:WAN mode - Red: No SIM/USIM card is found, the PIN is not verified, or the SIM/USIM card is not working properly. Failed to connect to a mobile network |  |  |
|  | Signal | One to three: Weak to Strong signal Off: out signal |  |  |
|  | WPS/WIFI | White Blink: WPS open White Steady On: 2.4G WiFi is opened Off: 2.4G WiFi is closed |  |  |
|  | LAN | On/Off |  |  |
|  | Power | On/Off |  |  |
| Button | Power switch, Reset switch, WPS switch |  |  |  |
| Antenna | - Built-in GSM/UMTS/LTE main diversity antenna - Built-in GSM/UMTS/LTE diversity antenna - Built-in WLAN antenna |  |  |  |
| Dimensions (D × W × H) | 180 mm x126 mm x38mm |  |  |  |
| Weight | about 226 g (Does not contain the power adapter) |  |  |  |
| Temperature | - Operating: 0℃ to +40℃ |  |  |  |
|  | - Storage: -20℃ to +70℃ |  |  |  |
| Humidity | 5% to 95% ( non-condensing) |  |  |  |

## Improvements in the Previous Version

| Index | Case ID | Issue Description |
| --- | --- | --- |
| NA |  |  |

## Known Limitations and Issues

| Index | Case ID | Issue Description |
| --- | --- | --- |
| NA |  |  |

# Firmware

## Version Description

| Firmware Version: | 21.311.03.02.141 |
| --- | --- |
| Baseline information | BalongV700R110C30B311 |
| OS | VxWorks 6.8+linux 3.4.5 |

## Firmware Specifications

| Item | Description |
| --- | --- |
| SMS | - Writing/Sending/Receiving - Sending/Receiving extra-long messages - Storage: Up to 500 messages can be saved in the internal memory - New message prompt |
| Network connection setup | - APN management: create, delete and edit. - Set up network connection |
| WLAN setup | - SSID broadcasting and hiding - Open system and shared key authentication - ASCII and HEX keys - 64/128-bit WEP encryption - 256-bit WPA-PSK and WPA2-PSK encryption - AES encryption algorithm - TKIP and AES integrated encryption algorithm - Automatic adjustment of ratios - Display STA status - WLAN MAC filter |
| Firewall setup | - Firewall Switch - LAN IP Filter - Virtual Server - DMZ Service |
| NAT setup | - CONE NAT - Symmetric NAT - ALG - VPN passthrough |
| DHCP setup | - DHCP server enabling and disabling - Address pool of the DHCP server setup - DHCP lease time setup |
| IPv6v4/IPv4 dual stack | DHCPv6/v4 server and client DNSv6/v4 server and client Display IPv6/v4 WAN address |
| Other | Network connection settings: - Automatic network selection and registration - Manual network selection and registration |
|  | Network status display: signal, operator name, system mode, and so on. |
|  | Selection of network connection types, for example: - Support LTE networks ON/OFF |
|  | PIN management: activate/deactivate PIN, PIN lock, changing PIN, unblocking by using the PUK. |
| System requirement | - Windows XP SP3, Windows Vista SP1/SP2, Windows 7, Windows 8 (does not support Windows RT) - Mac OS X 10.6, 10.7 and 10.8 with latest upgrades - Your computer’s hardware system should meet or exceed the recommended system requirements for the installed version of OS |

## Improvement in the Previous Version

| Index | Case ID | Issue Description |
| --- | --- | --- |
| 1 | translation ISSUE | If the product has not a Ethernet port, you need a LAN USB adapter” is not correctly translated into Romanian, it should be " Notă: Dacă produsul nu are un port Ethernet, aveţi nevoie de un adaptor LAN USB.” |
| 2 | translation ISSUE | translation strings changes as "values-ro" column |

## Known Limitations and Issues

| Index | Case ID | Issue Description |
| --- | --- | --- |
| 1 | Unrealized Features | NA |

# WebUI

## Version Description

| WebUI Version: | 17.100.09.00.03 |
| --- | --- |

## WebUI/HiLink Specifications

| Item | Specifications |
| --- | --- |

## Improvement in the Previous Version

| Index | Case ID | Issue Description |
| --- | --- | --- |
| NA |  |  |

## Known Limitations and Issues

| Index | Case ID | Issue Description |
| --- | --- | --- |
| NA |  |  |

# Software Vulnerabilities Fixes

| Software/Module name | Version | CVE ID | Vulnerability Description | Solution |
| --- | --- | --- | --- | --- |
| NA |  |  |  | Baseline already has finished developing with security issues, not relate to this one. |