| Product name | Confidentiality level |
| --- | --- |
| B310s-22 | CONFIDENTIAL |
| Product version | Total 14 pages |
| V1.0 |  |

B310s-22TCPU-V200R001B321D03SP00C738 Firmware Release Notes

| Prepared by | B310s-22 Team | Date | 2017-04-10 |
| --- | --- | --- | --- |

Huawei Technologies Co., Ltd.

All rights reserved

Revision Record

| Date | Revision version | FW-WebUI/HiLink Version | Change Description | Author |
| --- | --- | --- | --- | --- |
| 2017-4-10 | 1.0 | FW 21.321.03.00.738 | The 1th Version | B310s-22 Team |

Table of Contents

1	Main Features	4

2	Hardware	5

2.1	Version Description	5

2.2	Hardware Specifications	5

2.3	Improvements in the Previous Version	6

2.4	Known Limitations and Issues	7

3	Firmware	7

3.1	Version Description	7

3.2	Firmware Specifications	7

3.3	Improvement in the Previous Version	8

3.4	Known Limitations and Issues	9

4	WebUI	9

4.1	Version Description	9

4.2	WebUI/HiLink Specifications	9

4.3	Improvement in the Previous Version	9

4.4	Known Limitations and Issues	9

5	Software Vulnerabilities Fixes	9

B310s-22 Firmware Release Notes

| - Abbreviations | description |
| --- | --- |

# Main Features

The B310s-22 mainly supports the following features:

- LTE FDD (DL) data service of up to 150 Mbit/s

- LTE FDD (UL) data service of up to 50 Mbit/s

- LTE TDD (DL) data service of up to 112 Mbit/s

- LTE TDD (UL) data service of up to 10 Mbit/s

- DC-HSPA+ (DL) data service of up to 42 Mbit/s

- HSPA+ (DL) data service of up to 21.6 Mbit/s

- HSDPA (DL) data service of up to 14.4 Mbit/s

- HSUPA (UL) data service of up to 5.76 Mbit/s

- UMTS data service of up to 384 kbit/s

- EDGE data service of up to 236.8 kbit/s

- EDGE data service of download to 296 kbit/s

- GPRS data service of up to 85.6 kbit/s

- PS domain data service based on LTE/UMTS/GSM

- SMS based on CS/PS domain of GSM and UMTS, CS domain of LTE

- Wi-Fi

- Support for HUAWEI Mobile WiFi App

- Press and Play

- IPv6v4 /IPv4 dual stack

- Built-in DHCP Server, DNS RELAY and NAT

- Online software upgrade

- Traffic statistic

- LED indicators

- Built-in UMTS and WLAN high gain antenna LTE/GSM

- Windows XP SP3, Windows Vista SP1/SP2, Windows 7, Windows 8, Windows 8.1 (does not support Windows RT), MAC OS X 10.7, 10.8 and 10.9 with latest upgrades

# Hardware

## Version Description

| Hardware Version: | WL1B310FM03 (B310s-22) |
| --- | --- |
| Platform & Chipset: | Balong Hi6921 & AR 8035 |

## Hardware Specifications

| Item | Specifications |  |  |  |
| --- | --- | --- | --- | --- |
| Technical standard | - WAN: LTE/ DC-HSPA+/HSPA+/HSPA/UMTS/EDGE/GPRS/GSM |  |  |  |
|  | - WLAN: IEEE 802.11b/g/n |  |  |  |
| Operating frequency | LTE: B1/B3/B7/B8/B20/B38 |  |  |  |
|  | HSPA+/HSPA/UMTS: B1/B8 |  |  |  |
|  | EDGE/GPRS/GSM: 1900/1800/900/850 MHz |  |  |  |
|  | WLAN: 2.4 GHz |  |  |  |
| Internal memory | 512 MB Flash,256 MB Memory |  |  |  |
| Maximum transmitter power | UMTS: 24 (+1/-3) dBm |  |  |  |
|  | WLAN |  | 802.11b: 16 dBm |  |
|  |  |  | 802.11g: 17 dBm |  |
|  |  |  | 802.11n: 17 dBm |  |
| Receiver sensitivity | UMTS: Confirm to 3GPP Requirements |  |  |  |
|  | WLAN 802.11b |  |  | -76 dBm@11 Mbit/s |
|  |  |  |  | -82 dBm@1 Mbit/s |
|  | WLAN 802.11g: -65 dBm@54 Mbit/s |  |  |  |
|  | WLAN 802.11n: -64 dBm@65 Mbit/s |  |  |  |
| WLAN speed | 802.11b: Up to 11 Mbit/s |  |  |  |
|  | 802.11g: Up to 54 Mbit/s |  |  |  |
|  | 802.11n: HT40 MCS15(300Mbit/s), HT20 MCS15(144.4Mbit/s) |  |  |  |
| Maximum power consumption | 12 W |  |  |  |
| Power supply | - AC: 100–240 V |  |  |  |
|  | - DC: 12 V, 1 A |  |  |  |
| External interfaces | WAN/LAN: 1 RJ45,GE FXS:1 RJ11 |  |  |  |
|  | SIM card interface: standard 6-pin SIM card interface |  |  |  |
| Indicators | Mode: | cyan: 4G mode blue: 3G mode yellow: 2G mode green:WAN mode - Red: No SIM/USIM card is found, the PIN is not verified, or the SIM/USIM card is not working properly. Failed to connect to a mobile network |  |  |
|  | Signal | One to three: Weak to Strong signal Off: out signal |  |  |
|  | WPS/WIFI | White Blink: WPS open White Steady On: 2.4G WiFi is opened Off: 2.4G WiFi is closed |  |  |
|  | LAN | On/Off |  |  |
|  | Power | On/Off |  |  |
| Button | Power switch, Reset switch, WPS switch |  |  |  |
| Antenna | - Built-in GSM/UMTS/LTE main diversity antenna - Built-in GSM/UMTS/LTE diversity antenna - Built-in WLAN antenna |  |  |  |
| Dimensions (D × W × H) | 180 mm x126 mm x38mm |  |  |  |
| Weight | about 226 g (Does not contain the power adapter) |  |  |  |
| Temperature | - Operating: 0℃ to +40℃ |  |  |  |
|  | - Storage: -20℃ to +70℃ |  |  |  |
| Humidity | 5% to 95% ( non-condensing) |  |  |  |

## Improvements in the Previous Version

| Index | Case ID | Issue Description |
| --- | --- | --- |
| NA |  |  |

## Known Limitations and Issues

| Index | Case ID | Issue Description |
| --- | --- | --- |
| NA |  |  |

# Firmware

## Version Description

| Firmware Version: | 21.321.03.00.738 |
| --- | --- |
| Baseline information | BalongV700R110C30B321 |
| OS | VxWorks 6.8+linux 3.4.5 |

## Firmware Specifications

| Item | Description |
| --- | --- |
| Authentication of WIFI | - Authentication of WiFi: WPA/WPA2-PSK - Encription of WiFi: TKIP + AES - WLAN Country: All |
| Cs Voice | - support |
| Network Mode | - Display network mode list as below: - Auto - 4G only - 3G only - 3G/2G only - 2G only |
| HTTPS remote management through WAN interface | - HTTPS remote management with port 61443 |
| https remote access | - 1)default enable - 2)no IP limitations - 3)customer can edit/remove this rule |
| CLIP Mode | - FSK as per ITU-T-V.23 - ETSI ETS 300 659-1 (2001), ETSI ETS 300 778-1 (1997) - 1300Hz ± 1.5% 2100Hz ± 1.5% 1200bps ±1% - Initial ringing signal 350-400 ms - FSK generating level -13.5 dBm± 1.5 dB, |
| WLAN and LAN DCHP Network Addresses | 192.168.1.2 - 192.168.1.100 |
| DHCP editable | - all of four parts of IP address can be edited(192.168.X.X ---> X.X.X.X) - the last two parts of subnet mask can be edited (255.255.255.0 ---> 255.255.Y.Y) |
| NAT Switch | 1.Add NAT Switch in NAT Settings page. there has two options: Enabled, Disabled. default as enabled. Refer to the right screenshot. 2.When set NAT switch as Disabled,pls also check the box of "Enable LAN to LAN ping pass WAN" in Firewall Switch page. ping LAN to LAN function as below: 1) add a select box,it can open or close the "Enable LAN to LAN ping pass WAN" function ; 2) add simple introduction：“Enable LAN to LAN ping pass WAN function will enable the outer LAN which is connected through WAN to ping LAN through local CPE.”； 3) when select this function ,the webUI need display a warning message: "WARNING! Kindly be informed that enabling this function will decrease the security of the CPE, which may allow outer interference. " 4) “Enable LAN to LAN ping pass WAN” function as below: the two LANs(LAN1 & LAN2) gateway also use B310s-22, need support PC1(in LAN1) ping PC2(in LAN2) enable; need support PC2(in LAN2) ping PC1(in LAN1) enable. 5) need support transfer TCP and UDP data between LAN1 and LAN2. |

## Improvement in the Previous Version

| Index | Case ID | Issue Description |
| --- | --- | --- |
| 1 |  | First custom |

## Known Limitations and Issues

| Index | Case ID | Issue Description |
| --- | --- | --- |
|  |  | NA |

# WebUI

## Version Description

| WebUI Version: | 17.100.09.00.03 |
| --- | --- |

## WebUI/HiLink Specifications

| Item | Specifications |
| --- | --- |

## Improvement in the Previous Version

| Index | Case ID | Issue Description |
| --- | --- | --- |
|  |  | First custom |

## Known Limitations and Issues

| Index | Case ID | Issue Description |
| --- | --- | --- |
|  |  | NA |

# Software Vulnerabilities Fixes

| Software/Module name | Version | CVE ID | Vulnerability Description | Solution |
| --- | --- | --- | --- | --- |