| Product name | Confidentiality level |
| --- | --- |
| B310s-927 | CONFIDENTIAL |
| Product version | Total 37 pages |
| V5.0 |  |

B310s-927 Firmware Release Notes

| Prepared by | B310s-927 Team | Date | 2017-02-17 |
| --- | --- | --- | --- |
| Reviewed by | B310s-927 Team | Date | 2017-02-17 |
| Approved by | B310s-927 Team | Date | 2017-02-17 |

Huawei Technologies Co., Ltd.

All rights reserved

Revision Record

| Date | Revision version | FW-WebUI/HiLink Version | Change Description | Author |
| --- | --- | --- | --- | --- |
| 2014-11-9 | 1.0 | FW 21.180.03.00.00 | The 1th Version | B310s-927 Team |
| 2015-2-4 | 2.0 | FW 21.300.01.00.00 | The 2th Version | B310s-927 Team |
| 2015-3-18 | 2.1 | FW 21.300.07.00.00 | The 3th Version | B310s-927 Team |
| 2015-5-14 | 3.0 | FW 21.311.03.00.00 | The 4th Version | B310s-927 Team |
| 2015-6-27 | 3.1 | FW 21.311.05.00.00 | The 5th Version | B310s-927 Team |
| 2015-9-16 | 4.0 | FW 21.313.01.00.00 | The 6th Version | B310s-927 Team |
| 2015-10-14 | 4.1 | FW 21.313.03.00.00 | The 7th Version | B310s-927 Team |
| 2015-11-22 | 4.2 | FW 21.313.05.00.00 | The 8h Version | B310s-927 Team |
| 2016-01-28 | 5.0 | FW 21.316.01.00.00 | The 9h Version | B310s-927 Team |
| 2016-05-28 | 6.0 | FW 21.318.01.00.00 | The 10h Version | B310s-927 Team |
| 2016-09-02 | 7.0 | FW 21.321.01.00.00 | The 11h Version | B310s-927 Team |
| 2016-09-17 | 7.1 | FW 21.321.03.00.00 | The 12h Version | B310s-927 Team |
| 2016-12-17 | 8.0 | FW 21.323.01.00.00 | The 13h Version | B310s-927 Team |
| 2017-02-17 | 8.1 | FW 21.323.03.00.00 | The 14h Version | B310s-927 Team |

Table of Contents

1	Main Features	4

2	Hardware	5

2.1	Version Description	5

2.2	Hardware Specifications	5

2.3	Improvements in the Previous Version	6

2.4	Known Limitations and Issues	7

3	Firmware	7

3.1	Version Description	7

3.2	Firmware Specifications	7

3.3	Improvement in the Previous Version	8

3.4	Known Limitations and Issues	9

4	WebUI	9

4.1	Version Description	9

4.2	WebUI/HiLink Specifications	9

4.3	Improvement in the Previous Version	9

4.4	Known Limitations and Issues	9

5	Software Vulnerabilities Fixes	9

B310s-927 Firmware Release Notes

| - Abbreviations | description |
| --- | --- |

# Main Features

The B310s-927 mainly supports the following features:

- LTE FDD (DL) data service of up to 150 Mbit/s

- LTE FDD (UL) data service of up to 50 Mbit/s

- LTE TDD (DL) data service of up to 112 Mbit/s

- LTE TDD (UL) data service of up to 10 Mbit/s

- DC-HSPA+ (DL) data service of up to 42 Mbit/s

- HSPA+ (DL) data service of up to 21.6 Mbit/s

- HSDPA (DL) data service of up to 14.4 Mbit/s

- HSUPA (UL) data service of up to 5.76 Mbit/s

- UMTS data service of up to 384 kbit/s

- EDGE data service of up to 236.8 kbit/s

- EDGE data service of download to 296 kbit/s

- GPRS data service of up to 85.6 kbit/s

- PS domain data service based on LTE/UMTS/GSM

- SMS based on CS/PS domain of GSM and UMTS, CS domain of LTE

- Wi-Fi

- Support for HUAWEI Mobile WiFi App

- Press and Play

- Built-in DHCP Server, DNS RELAY and NAT

- Online software upgrade

- Traffic statistic

- LED indicators

- Built-in UMTS and WLAN high gain antenna LTE/GSM

- Windows XP SP3, Windows Vista SP1/SP2, Windows 7, Windows 8, Windows 8.1 (does not support Windows RT), MAC OS X 10.7, 10.8 and 10.9 with latest upgrades

# Hardware

## Version Description

| Hardware Version: | WL1B310TM (B310s-927) |
| --- | --- |
| Platform & Chipset: | Balong Hi6921 & AR 8035 |

## Hardware Specifications

| Item | Specifications |  |  |  |
| --- | --- | --- | --- | --- |
| Technical standard | - WAN: LTE/ DC-HSPA+/HSPA+/HSPA/UMTS/EDGE/GPRS/GSM |  |  |  |
|  | - WLAN: IEEE 802.11b/g/n |  |  |  |
| Operating frequency | LTE: B3/B40 |  |  |  |
|  | HSPA+/HSPA/UMTS: B1(2100) /B8(900) MHz |  |  |  |
|  | EDGE/GPRS/GSM: 1900/1800/900/850 MHz |  |  |  |
|  | WLAN: 2.4 GHz |  |  |  |
| Internal memory | 512 MB Flash,256 MB Memory |  |  |  |
| Maximum transmitter power | UMTS: 24 (+1/-3) dBm |  |  |  |
|  | WLAN |  | 802.11b: 19 dBm |  |
|  |  |  | 802.11g: 17 dBm |  |
|  |  |  | 802.11n: 17 dBm |  |
| Receiver sensitivity | UMTS: Confirm to 3GPP Requirements |  |  |  |
|  | WLAN 802.11b |  |  | -76 dBm@11 Mbit/s |
|  |  |  |  | -82 dBm@1 Mbit/s |
|  | WLAN 802.11g: -65 dBm@54 Mbit/s |  |  |  |
|  | WLAN 802.11n: -64 dBm@65 Mbit/s |  |  |  |
| WLAN speed | 802.11b: Up to 11 Mbit/s |  |  |  |
|  | 802.11g: Up to 54 Mbit/s |  |  |  |
|  | 802.11n: HT40 MCS15(300Mbit/s), HT20 MCS15(144.4Mbit/s) |  |  |  |
| Maximum power consumption | 12 W |  |  |  |
| Power supply | - AC: 100–240 V |  |  |  |
|  | - DC: 12 V, 1 A |  |  |  |
| External interfaces | WAN/LAN: 1 RJ45,GE FXS:1 RJ11 |  |  |  |
|  | SIM card interface: standard 6-pin SIM card interface |  |  |  |
| Indicators | Mode: | cyan: 4G mode blue: 3G mode yellow: 2G mode green:WAN mode - Red: No SIM/USIM card is found, the PIN is not verified, or the SIM/USIM card is not working properly. Failed to connect to a mobile network |  |  |
|  | Signal | One to three: Weak to Strong signal Off: out signal |  |  |
|  | WPS/WIFI | White Blink: WPS open White Steady On: 2.4G WiFi is opened Off: 2.4G WiFi is closed |  |  |
|  | LAN | On/Off |  |  |
|  | Power | On/Off |  |  |
| Button | Power switch, Reset switch, WPS switch |  |  |  |
| Antenna | - Built-in GSM/UMTS/LTE main diversity antenna - Built-in GSM/UMTS/LTE diversity antenna - Built-in WLAN antenna |  |  |  |
| Dimensions (D × W × H) | 180 mm x126 mm x38mm |  |  |  |
| Weight | about 226 g (Does not contain the power adapter) |  |  |  |
| Temperature | - Operating: 0℃ to +40℃ |  |  |  |
|  | - Storage: -20℃ to +70℃ |  |  |  |
| Humidity | 5% to 95% ( non-condensing) |  |  |  |

## Improvements in the Previous Version

| Index | Case ID | Issue Description |
| --- | --- | --- |
| NA |  |  |

## Known Limitations and Issues

| Index | Case ID | Issue Description |
| --- | --- | --- |
| NA |  |  |

# Firmware

## Version Description

| Firmware Version: | 21.323.03.00.1065 |
| --- | --- |
| Baseline information : | BalongV700R110C30B323 |
| OS | VxWorks 6.8+linux 3.4.5 |

## Firmware Specifications

| Item | Description |
| --- | --- |
| SMS | - Writing/Sending/Receiving - Sending/Receiving extra-long messages - Storage: Up to 500 messages can be saved in the internal memory - New message prompt |
| Network connection setup | - APN management: create, delete and edit. - Set up network connection |
| WLAN setup | - SSID broadcasting and hiding - Open system and shared key authentication - ASCII and HEX keys - 64/128-bit WEP encryption - 256-bit WPA-PSK and WPA2-PSK encryption - AES encryption algorithm - TKIP and AES integrated encryption algorithm - Automatic adjustment of ratios - Display STA status - WLAN MAC filter |
| Firewall setup | - Firewall Switch - LAN IP Filter - Virtual Server - DMZ Service |
| NAT setup | - CONE NAT - Symmetric NAT - ALG - VPN passthrough |
| DHCP setup | - DHCP server enabling and disabling - Address pool of the DHCP server setup - DHCP lease time setup |
| IPv6v4/IPv4 dual stack | DHCPv6/v4 server and client DNSv6/v4 server and client Display IPv6/v4 WAN address |
| Other | Network connection settings: - Automatic network selection and registration - Manual network selection and registration |
|  | Network status display: signal, operator name, system mode, and so on. |
|  | Selection of network connection types, for example: - Support LTE networks ON/OFF |
|  | PIN management: activate/deactivate PIN, PIN lock, changing PIN, unblocking by using the PUK. |
| System requirement | - Windows XP SP3, Windows Vista SP1/SP2, Windows 7, Windows 8 (does not support Windows RT) - Mac OS X 10.6, 10.7 and 10.8 with latest upgrades - Your computer’s hardware system should meet or exceed the recommended system requirements for the installed version of OS |

## Improvement in the Previous Version

| Index | Case ID | Issue Description |
| --- | --- | --- |

## Known Limitations and Issues

| Index | Case ID | Issue Description |
| --- | --- | --- |

# WebUI

## Version Description

| WebUI Version: | 17.100.09.00.03 |
| --- | --- |

## WebUI/HiLink Specifications

| Item | Specifications |
| --- | --- |

## Improvement in the Previous Version

| Index | Case ID | Issue Description |
| --- | --- | --- |

## Known Limitations and Issues

| Index | Case ID | Issue Description |
| --- | --- | --- |

# Software Vulnerabilities Fixes

| Software/Module name | Version | CVE ID | Vulnerability Description | Solution |
| --- | --- | --- | --- | --- |
| Portable UPnP SDK | LibUPnP 1.6.12 | CVE-2012-5960 | Stack-based buffer overflow in the unique_service_name function in ssdp/ssdp_server.c in the SSDP parser in the portable SDK for UPnP Devices (aka libupnp, formerly the Intel SDK for UPnP devices) before 1.6.18 allows remote attackers to execute arbitrary code via a long UDN (aka upnp:rootdevice) field in a UDP packet. | Add memory protection check for errors .Refer to: https://cve.mitre.org/cgi-bin/cvename.cgi?name=CVE-2012-5960 |
| Portable UPnP SDK | LibUPnP 1.6.12 | CVE-2012-5959 | Stack-based buffer overflow in the unique_service_name function in ssdp/ssdp_server.c in the SSDP parser in the portable SDK for UPnP Devices (aka libupnp, formerly the Intel SDK for UPnP devices) before 1.6.18 allows remote attackers to execute arbitrary code via a long UDN (aka uuid) field within a string that contains a :: (colon colon) in a UDP packet. | Add memory protection check for errors .Refer to: https://cve.mitre.org/cgi-bin/cvename.cgi?name=CVE-2012-5959 |
| Portable UPnP SDK | LibUPnP 1.6.12 | CVE-2012-5958 | Stack-based buffer overflow in the unique_service_name function in ssdp/ssdp_server.c in the SSDP parser in the portable SDK for UPnP Devices (aka libupnp, formerly the Intel SDK for UPnP devices) before 1.6.18 allows remote attackers to execute arbitrary code via a UDP packet with a crafted string that is not properly handled after a certain pointer subtraction. | Add memory protection check for errors .Refer to: https://cve.mitre.org/cgi-bin/cvename.cgi?name=CVE-2012-5958 |
| Samba | 3.0.37 | CVE-2013-4475 | Samba 3.2.x through 3.6.x before 3.6.20, 4.0.x before 4.0.11, and 4.1.x before 4.1.1, when vfs_streams_depot or vfs_streams_xattr is enabled, allows remote attackers to bypass intended file restrictions by leveraging ACL differences between a file and an associated alternate data stream (ADS). | Don’t involve closing.Refer to the Samba website corresponding vulnerability, the problems in the Samba3.2.0 version, the current version is 3.0.37,refer：http://www.samba.org/samba/security/CVE-2013-4475 |
| Samba | 3.0.37 | CVE-2013-4124 | Integer overflow in the read_nttrans_ea_list function in nttrans.c in smbd in Samba 3.x before 3.5.22, 3.6.x before 3.6.17, and 4.x before 4.0.8 allows remote attackers to cause a denial of service (memory consumption) via a malformed packet. | https://ftp.samba.org/pub/samba/patches/security/samba-4.0.7-CVE-2013-4124.patch |
| Samba | 3.0.37 | CVE-2013-0454 | The SMB2 implementation in Samba 3.6.x before 3.6.6, as used on the IBM Storwize V7000 Unified 1.3 before 1.3.2.3 and 1.4 before 1.4.0.1 and possibly other products, does not properly enforce CIFS share attributes, which allows remote authenticated users to (1) write to a read-only share; (2) trigger data-integrity problems related to the oplock, locking, coherency, or leases attribute; or (3) have an unspecified impact by leveraging incorrect handling of the browseable or "hide unreadable" parameter. | https://ftp.samba.org/pub/samba/patches/security/samba-3.6-CVE-2013-0454.patch |
| Samba | 3.0.37 | CVE-2013-0214 | Cross-site request forgery (CSRF) vulnerability in the Samba Web Administration Tool (SWAT) in Samba 3.x before 3.5.21, 3.6.x before 3.6.12, and 4.x before 4.0.2 allows remote attackers to hijack the authentication of arbitrary users by leveraging knowledge of a password and composing requests that perform SWAT actions. | https://download.samba.org/pub/samba/patches/security/samba-3.5.20-CVE-2013-0213-CVE-2013-0214.patch |
| Samba | 3.0.37 | CVE-2013-0213 | The Samba Web Administration Tool (SWAT) in Samba 3.x before 3.5.21, 3.6.x before 3.6.12, and 4.x before 4.0.2 allows remote attackers to conduct clickjacking attacks via a (1) FRAME or (2) IFRAME element. | https://download.samba.org/pub/samba/patches/security/samba-3.5.20-CVE-2013-0213-CVE-2013-0214.patch |
| Samba | 3.0.37 | CVE-2012-1182 | The RPC code generator in Samba 3.x before 3.4.16, 3.5.x before 3.5.14, and 3.6.x before 3.6.4 does not implement validation of an array length in a manner consistent with validation of array memory allocation, which allows remote attackers to execute arbitrary code via a crafted RPC call. | https://download.samba.org/pub/samba/patches/security/samba-3.0.37-CVE-2012-1182.patch |
| Samba | 3.0.37 | CVE-2011-2724 | The check_mtab function in client/mount.cifs.c in mount.cifs in smbfs in Samba 3.5.10 and earlier does not properly verify that the (1) device name and (2) mountpoint strings are composed of valid characters, which allows local users to cause a denial of service (mtab corruption) via a crafted string. NOTE: this vulnerability exists because of an incorrect fix for CVE-2010-0547. | Don’t involve closing.Refer to：https://cve.mitre.org/cgi-bin/cvename.cgi?name=CVE-2011-2724 |
| Samba | 3.0.37 | CVE-2011-2694 | Cross-site scripting (XSS) vulnerability in the chg_passwd function in web/swat.c in the Samba Web Administration Tool (SWAT) in Samba 3.x before 3.5.10 allows remote authenticated administrators to inject arbitrary web script or HTML via the username parameter to the passwd program (aka the user field to the Change Password page). | https://download.samba.org/pub/samba/patches/security/samba-3.3.15-CVE-2011-2694.patch |
| Samba | 3.0.37 | CVE-2011-2522 | Multiple cross-site request forgery (CSRF) vulnerabilities in the Samba Web Administration Tool (SWAT) in Samba 3.x before 3.5.10 allow remote attackers to hijack the authentication of administrators for requests that (1) shut down daemons, (2) start daemons, (3) add shares, (4) remove shares, (5) add printers, (6) remove printers, (7) add user accounts, or (8) remove user accounts, as demonstrated by certain start, stop, and restart parameters to the status program. | https://download.samba.org/pub/samba/patches/security/samba-3.3.15-CVE-2011-2522.patch |
| Samba | 3.0.37 | CVE-2011-1678 | smbfs in Samba 3.5.8 and earlier attempts to use (1) mount.cifs to append to the /etc/mtab file and (2) umount.cifs to append to the /etc/mtab.tmp file without first checking whether resource limits would interfere, which allows local users to trigger corruption of the /etc/mtab file via a process with a small RLIMIT_FSIZE value, a related issue to CVE-2011-1089. | Don’t involve closing.,/etc is a read-only file can not be tampered with. Refer to：https://cve.mitre.org/cgi-bin/cvename.cgi?name=CVE-2011-1678 |
| Samba | 3.0.37 | CVE-2011-0719 | Samba 3.x before 3.3.15, 3.4.x before 3.4.12, and 3.5.x before 3.5.7 does not perform range checks for file descriptors before use of the FD_SET macro, which allows remote attackers to cause a denial of service (stack memory corruption, and infinite loop or daemon crash) by opening a large number of files, related to (1) Winbind or (2) smbd. | https://download.samba.org/pub/samba/patches/security/samba-3.3.14-CVE-2011-0719.patch |
| Samba | 3.0.37 | CVE-2010-3069 | Stack-based buffer overflow in the (1) sid_parse and (2) dom_sid_parse functions in Samba before 3.5.5 allows remote attackers to cause a denial of service (crash) and possibly execute arbitrary code via a crafted Windows Security ID (SID) on a file share. | https://download.samba.org/pub/samba/patches/security/samba-3.3.13-CVE-2010-3069.patch |
| Samba | 3.0.37 | CVE-2010-2063 | Buffer overflow in the SMB1 packet chaining implementation in the chain_reply function in process.c in smbd in Samba 3.0.x before 3.3.13 allows remote attackers to cause a denial of service (memory corruption and daemon crash) or possibly execute arbitrary code via a crafted field in a packet. | https://download.samba.org/pub/samba/patches/security/samba-3.0.37-CVE-2010-2063.patch |
| Samba | 3.0.37 | CVE-2010-1642 | The reply_sesssetup_and_X_spnego function in sesssetup.c in smbd in Samba before 3.4.8 and 3.5.x before 3.5.2 allows remote attackers to trigger an out-of-bounds read, and cause a denial of service (process crash), via a \xff\xff security blob length in a Session Setup AndX request. | https://git.samba.org/?p=samba.git;a=commit;h=9280051bfba337458722fb157f3082f93cbd9f2b |
| Samba | 3.0.37 | CVE-2010-1635 | The chain_reply function in process.c in smbd in Samba before 3.4.8 and 3.5.x before 3.5.2 allows remote attackers to cause a denial of service (NULL pointer dereference and process crash) via a Negotiate Protocol request with a certain 0x0003 field value followed by a Session Setup AndX request with a certain 0x8003 field value. | Don’t involve closing.There is no problem of output has been done to determine.Refer to: https://cve.mitre.org/cgi-bin/cvename.cgi?name=CVE-2010-1635 |
| Samba | 3.0.37 | CVE-2010-0547 | client/mount.cifs.c in mount.cifs in smbfs in Samba 3.4.5 and earlier does not verify that the (1) device name and (2) mountpoint strings are composed of valid characters, which allows local users to cause a denial of service (mtab corruption) via a crafted string. | Don’t involve closing .Function problems do not exist without treatment. Refer to https://cve.mitre.org/cgi-bin/cvename.cgi?name=CVE-2010-0547 |
| Samba | 3.0.37 | CVE-2012-6150 | The winbind_name_list_to_sid_string_list function in nsswitch/pam_winbind.c in Samba through 4.1.2 handles invalid require_membership_of group names by accepting authentication by any user, which allows remote authenticated users to bypass intended access restrictions in opportunistic circumstances by leveraging an administrator's pam_winbind configuration-file mistake. | Don’t involve closing .Refer to the Samba website corresponding vulnerability to explain the problem, in the 3.3.10, 3.4.3, 3.5.0 and later Later, the current version is 3.0.37, the specific reference：http://www.samba.org/samba/security/CVE-2012-6150 |
| Samba | 3.0.37 | CVE-2013-4408 | Heap-based buffer overflow in the dcerpc_read_ncacn_packet_done function in librpc/rpc/dcerpc_util.c in winbindd in Samba 3.x before 3.6.22, 4.0.x before 4.0.13, and 4.1.x before 4.1.3 allows remote AD domain controllers to execute arbitrary code via an invalid fragment length in a DCE-RPC packet. | Don’t involve closing.The current version does not have this function, do not need to deal with. Refer to https://cve.mitre.org/cgi-bin/cvename.cgi?name=CVE-2013-4408 |
| Openssl | 0.98y | CVE-2014-3470 | The ssl3_send_client_key_exchange function in s3_clnt.c in OpenSSL before 0.9.8za, 1.0.0 before 1.0.0m, and 1.0.1 before 1.0.1h, when an anonymous ECDH cipher suite is used, allows remote attackers to cause a denial of service (NULL pointer dereference and client crash) by triggering a NULL certificate value. | https://git.openssl.org/gitweb/?p=openssl.git;a=commit;h=8011cd56e39a433b1837465259a9bd24a38727fb |
| Openssl | 1.0.0a | CVE-2014-3470 | The ssl3_send_client_key_exchange function in s3_clnt.c in OpenSSL before 0.9.8za, 1.0.0 before 1.0.0m, and 1.0.1 before 1.0.1h, when an anonymous ECDH cipher suite is used, allows remote attackers to cause a denial of service (NULL pointer dereference and client crash) by triggering a NULL certificate value. | https://git.openssl.org/gitweb/?p=openssl.git;a=commit;h=8011cd56e39a433b1837465259a9bd24a38727fb |
| Openssl | 1.0.1e | CVE-2014-3470 | The ssl3_send_client_key_exchange function in s3_clnt.c in OpenSSL before 0.9.8za, 1.0.0 before 1.0.0m, and 1.0.1 before 1.0.1h, when an anonymous ECDH cipher suite is used, allows remote attackers to cause a denial of service (NULL pointer dereference and client crash) by triggering a NULL certificate value. | https://git.openssl.org/gitweb/?p=openssl.git;a=commit;h=8011cd56e39a433b1837465259a9bd24a38727fb |
| Openssl | 0.98y | CVE-2014-0224 | OpenSSL before 0.9.8za, 1.0.0 before 1.0.0m, and 1.0.1 before 1.0.1h does not properly restrict processing of ChangeCipherSpec messages, which allows man-in-the-middle attackers to trigger use of a zero-length master key in certain OpenSSL-to-OpenSSL communications, and consequently hijack sessions or obtain sensitive information, via a crafted TLS handshake, aka the "CCS Injection" vulnerability. | https://git.openssl.org/gitweb/?p=openssl.git;a=commit;h=bc8923b1ec9c467755cd86f7848c50ee8812e441 |
| Openssl | 1.0.0a | CVE-2014-0224 | OpenSSL before 0.9.8za, 1.0.0 before 1.0.0m, and 1.0.1 before 1.0.1h does not properly restrict processing of ChangeCipherSpec messages, which allows man-in-the-middle attackers to trigger use of a zero-length master key in certain OpenSSL-to-OpenSSL communications, and consequently hijack sessions or obtain sensitive information, via a crafted TLS handshake, aka the "CCS Injection" vulnerability. | https://git.openssl.org/gitweb/?p=openssl.git;a=commit;h=bc8923b1ec9c467755cd86f7848c50ee8812e441 |
| Openssl | 1.0.1e | CVE-2014-0224 | OpenSSL before 0.9.8za, 1.0.0 before 1.0.0m, and 1.0.1 before 1.0.1h does not properly restrict processing of ChangeCipherSpec messages, which allows man-in-the-middle attackers to trigger use of a zero-length master key in certain OpenSSL-to-OpenSSL communications, and consequently hijack sessions or obtain sensitive information, via a crafted TLS handshake, aka the "CCS Injection" vulnerability. | https://git.openssl.org/gitweb/?p=openssl.git;a=commit;h=bc8923b1ec9c467755cd86f7848c50ee8812e441 |
| Openssl | 0.98y | CVE-2014-0221 | The dtls1_get_message_fragment function in d1_both.c in OpenSSL before 0.9.8za, 1.0.0 before 1.0.0m, and 1.0.1 before 1.0.1h allows remote attackers to cause a denial of service (recursion and client crash) via a DTLS hello message in an invalid DTLS handshake. | https://git.openssl.org/gitweb/?p=openssl.git;a=commit;h=d3152655d5319ce883c8e3ac4b99f8de4c59d846 |
| Openssl | 1.0.0a | CVE-2014-0221 | The dtls1_get_message_fragment function in d1_both.c in OpenSSL before 0.9.8za, 1.0.0 before 1.0.0m, and 1.0.1 before 1.0.1h allows remote attackers to cause a denial of service (recursion and client crash) via a DTLS hello message in an invalid DTLS handshake. | https://git.openssl.org/gitweb/?p=openssl.git;a=commit;h=d3152655d5319ce883c8e3ac4b99f8de4c59d846 |
| Openssl | 1.0.1e | CVE-2014-0221 | The dtls1_get_message_fragment function in d1_both.c in OpenSSL before 0.9.8za, 1.0.0 before 1.0.0m, and 1.0.1 before 1.0.1h allows remote attackers to cause a denial of service (recursion and client crash) via a DTLS hello message in an invalid DTLS handshake. | https://git.openssl.org/gitweb/?p=openssl.git;a=commit;h=d3152655d5319ce883c8e3ac4b99f8de4c59d846 |
| Openssl | 0.98y | CVE-2014-0198 | The do_ssl3_write function in s3_pkt.c in OpenSSL 1.x through 1.0.1g, when SSL_MODE_RELEASE_BUFFERS is enabled, does not properly manage a buffer pointer during certain recursive calls, which allows remote attackers to cause a denial of service (NULL pointer dereference and application crash) via vectors that trigger an alert condition. | http://people.canonical.com/~ubuntu-security/cve/2014/CVE-2014-0198.html |
| Openssl | 1.0.0a | CVE-2014-0198 | The do_ssl3_write function in s3_pkt.c in OpenSSL 1.x through 1.0.1g, when SSL_MODE_RELEASE_BUFFERS is enabled, does not properly manage a buffer pointer during certain recursive calls, which allows remote attackers to cause a denial of service (NULL pointer dereference and application crash) via vectors that trigger an alert condition. | http://people.canonical.com/~ubuntu-security/cve/2014/CVE-2014-0198.html |
| Openssl | 1.0.1e | CVE-2014-0198 | The do_ssl3_write function in s3_pkt.c in OpenSSL 1.x through 1.0.1g, when SSL_MODE_RELEASE_BUFFERS is enabled, does not properly manage a buffer pointer during certain recursive calls, which allows remote attackers to cause a denial of service (NULL pointer dereference and application crash) via vectors that trigger an alert condition. | http://people.canonical.com/~ubuntu-security/cve/2014/CVE-2014-0198.html |
| Openssl | 0.98y | CVE-2014-0195 | The dtls1_reassemble_fragment function in d1_both.c in OpenSSL before 0.9.8za, 1.0.0 before 1.0.0m, and 1.0.1 before 1.0.1h does not properly validate fragment lengths in DTLS ClientHello messages, which allows remote attackers to execute arbitrary code or cause a denial of service (buffer overflow and application crash) via a long non-initial fragment. | https://git.openssl.org/gitweb/?p=openssl.git;a=commit;h=1632ef744872edc2aa2a53d487d3e79c965a4ad3 |
| Openssl | 1.0.0a | CVE-2014-0195 | The dtls1_reassemble_fragment function in d1_both.c in OpenSSL before 0.9.8za, 1.0.0 before 1.0.0m, and 1.0.1 before 1.0.1h does not properly validate fragment lengths in DTLS ClientHello messages, which allows remote attackers to execute arbitrary code or cause a denial of service (buffer overflow and application crash) via a long non-initial fragment. | https://git.openssl.org/gitweb/?p=openssl.git;a=commit;h=1632ef744872edc2aa2a53d487d3e79c965a4ad3 |
| Openssl | 1.0.1e | CVE-2014-0195 | The dtls1_reassemble_fragment function in d1_both.c in OpenSSL before 0.9.8za, 1.0.0 before 1.0.0m, and 1.0.1 before 1.0.1h does not properly validate fragment lengths in DTLS ClientHello messages, which allows remote attackers to execute arbitrary code or cause a denial of service (buffer overflow and application crash) via a long non-initial fragment. | https://git.openssl.org/gitweb/?p=openssl.git;a=commit;h=1632ef744872edc2aa2a53d487d3e79c965a4ad3 |
| Openssl | 0.98y | CVE-2014-0076 | The Montgomery ladder implementation in OpenSSL through 1.0.0l does not ensure that certain swap operations have a constant-time behavior, which makes it easier for local users to obtain ECDSA nonces via a FLUSH+RELOAD cache side-channel attack. | http://git.openssl.org/gitweb/?p=openssl.git;a=commit;h=2198be3483259de374f91e57d247d0fc667aef29 |
| Openssl | 0.98y | CVE-2014-3512 | Multiple buffer overflows in crypto/srp/srp_lib.c in the SRP implementation in OpenSSL 1.0.1 before 1.0.1i allow remote attackers to cause a denial of service (application crash) or possibly have unspecified other impact via an invalid SRP (1) g, (2) A, or (3) B parameter. | https://git.openssl.org/gitweb/?p=openssl.git;a=commit;h=4a23b12a031860253b58d503f296377ca076427b |
| Openssl | 0.98y | CVE-2013-6450 | The DTLS retransmission implementation in OpenSSL 1.0.0 before 1.0.0l and 1.0.1 before 1.0.1f does not properly maintain data structures for digest and encryption contexts, which might allow man-in-the-middle attackers to trigger the use of a different context and cause a denial of service (application crash) by interfering with packet delivery, related to ssl/d1_both.c and ssl/t1_enc.c. | http://git.openssl.org/gitweb/?p=openssl.git;a=commit;h=34628967f1e65dc8f34e000f0f5518e21afbfc7b |
| Samba | 3.0.37 | CVE-2013-4496 | Samba 3.x before 3.6.23, 4.0.x before 4.0.16, and 4.1.x before 4.1.6 does not enforce the password-guessing protection mechanism for all interfaces, which makes it easier for remote attackers to obtain access via brute-force ChangePasswordUser2 (1) SAMR or (2) RAP attempts. | Don’t involve closing .CVE-2013-4496 vulnerability exists in the 3.4.0 version, the version is 3.0.37, without the need to merge. Https://www.samba.org/samba/security/CVE-2013-4496 Later, the current version is 3.0.37, the specific reference:http://www.samba.org/samba/security/CVE-2012-6150 |
| iptables | 1.4.0 | CVE-2012-2663 | extensions/libxt_tcp.c in iptables through 1.4.21 does not match TCP SYN+FIN packets in --syn rules, which might allow remote attackers to bypass intended firewall restrictions via crafted packets. NOTE: the CVE-2012-6638 fix makes this issue less relevant. | Don’t involve closing .he influence of CVE-2012-2663 kernel version of the Linux kernel 2.6.x, the official website address access to modify the kernel code, the EUAP code of Linux kernel code, and do not call `iptables -m TCP --syn command parameter, so no need to merge. Specific reference:http://git.kernel.org/cgit/linux/kernel/git/davem/net-next.git/commit/?id=fdf5af0daf8019cec2396cdef8fb042d80fe71fa |
| CUPS | 1.6.1 | CVE-2014-2856 | Cross-site scripting (XSS) vulnerability in scheduler/client.c in Common Unix Printing System (CUPS) before 1.7.2 allows remote attackers to inject arbitrary web script or HTML via the URL path, related to the is_path_absolute function. | http://www.cups.org/strfiles.php/3268/str4356.patch |
| Openssl | 0.98y | CVE-2010-5298 | Race condition in the ssl3_read_bytes function in s3_pkt.c in OpenSSL through 1.0.1g, when SSL_MODE_RELEASE_BUFFERS is enabled, allows remote attackers to inject data across sessions or cause a denial of service (use-after-free and parsing error) via an SSL connection in a multithreaded environment. | http://ftp.openbsd.org/pub/OpenBSD/patches/5.5/common/004_openssl.patch.sig |
| Openssl | 1.0.1e | CVE- 2014-0195 | The dtls1_reassemble_fragment function in d1_both.c in OpenSSL before 0.9.8za, 1.0.0 before 1.0.0m, and 1.0.1 before 1.0.1h does not properly validate fragment lengths in DTLS ClientHello messages, which allows remote attackers to execute arbitrary code or cause a denial of service (buffer overflow and application crash) via a long non-initial fragment. | https://git.openssl.org/gitweb/?p=openssl.git;a=commit;h=1632ef744872edc2aa2a53d487d3e79c965a4ad3 |
| Openssl | 1.0.1e | CVE-2010-5298 | Race condition in the ssl3_read_bytes function in s3_pkt.c in OpenSSL through 1.0.1g, when SSL_MODE_RELEASE_BUFFERS is enabled, allows remote attackers to inject data across sessions or cause a denial of service (use-after-free and parsing error) via an SSL connection in a multithreaded environment. | http://ftp.openbsd.org/pub/OpenBSD/patches/5.5/common/004_openssl.patch.sig |
| Openssl | 1.0.1e | CVE-2014-0076 | The Montgomery ladder implementation in OpenSSL through 1.0.0l does not ensure that certain swap operations have a constant-time behavior, which makes it easier for local users to obtain ECDSA nonces via a FLUSH+RELOAD cache side-channel attack. | https://git.openssl.org/gitweb/?p=openssl.git;a=commit;h=2198be3483259de374f91e57d247d0fc667aef29 |
| Openssl | 1.0.1e | CVE-2014-3505 | Double free vulnerability in d1_both.c in the DTLS implementation in OpenSSL 0.9.8 before 0.9.8zb, 1.0.0 before 1.0.0n, and 1.0.1 before 1.0.1i allows remote attackers to cause a denial of service (application crash) via crafted DTLS packets that trigger an error condition. | https://git.openssl.org/gitweb/?p=openssl.git;a=commit;h=bff1ce4e6a1c57c3d0a5f9e4f85ba6385fccfe8b |
| Openssl | 1.0.1e | CVE-2014-3506 | d1_both.c in the DTLS implementation in OpenSSL 0.9.8 before 0.9.8zb, 1.0.0 before 1.0.0n, and 1.0.1 before 1.0.1i allows remote attackers to cause a denial of service (memory consumption) via crafted DTLS handshake messages that trigger memory allocations corresponding to large length values. | https://git.openssl.org/gitweb/?p=openssl.git;a=commit;h=1250f12613b61758675848f6600ebd914ccd7636 |
| Openssl | 1.0.1e | CVE-2014-3507 | Memory leak in d1_both.c in the DTLS implementation in OpenSSL 0.9.8 before 0.9.8zb, 1.0.0 before 1.0.0n, and 1.0.1 before 1.0.1i allows remote attackers to cause a denial of service (memory consumption) via zero-length DTLS fragments that trigger improper handling of the return value of a certain insert function. | https://git.openssl.org/gitweb/?p=openssl.git;a=commit;h=d0a4b7d1a2948fce38515b8d862f43e7ba0ebf74 |
| Openssl | 1.0.1e | CVE-2014-3508 | The OBJ_obj2txt function in crypto/objects/obj_dat.c in OpenSSL 0.9.8 before 0.9.8zb, 1.0.0 before 1.0.0n, and 1.0.1 before 1.0.1i, when pretty printing is used, does not ensure the presence of '\0' characters, which allows context-dependent attackers to obtain sensitive information from process stack memory by reading output from X509_name_oneline, X509_name_print_ex, and unspecified other functions. | https://git.openssl.org/gitweb/?p=openssl.git;a=commit;h=0042fb5fd1c9d257d713b15a1f45da05cf5c1c87 |
| Openssl | 1.0.1e | CVE-2014-3510 | The ssl3_send_client_key_exchange function in s3_clnt.c in OpenSSL 0.9.8 before 0.9.8zb, 1.0.0 before 1.0.0n, and 1.0.1 before 1.0.1i allows remote DTLS servers to cause a denial of service (NULL pointer dereference and client application crash) via a crafted handshake message in conjunction with a (1) anonymous DH or (2) anonymous ECDH ciphersuite. | https://git.openssl.org/gitweb/?p=openssl.git;a=commit;h=17160033765480453be0a41335fa6b833691c049 |
| Openssl | 1.0.1e | CVE-2014-5139 | The ssl_set_client_disabled function in t1_lib.c in OpenSSL 1.0.1 before 1.0.1i allows remote SSL servers to cause a denial of service (NULL pointer dereference and client application crash) via a ServerHello message that includes an SRP ciphersuite without the required negotiation of that ciphersuite with the client. | https://git.openssl.org/gitweb/?p=openssl.git;a=commit;h=80bd7b41b30af6ee96f519e629463583318de3b0 |
| Openssl | 1.0.1e | CVE-2014-3512 | Multiple buffer overflows in crypto/srp/srp_lib.c in the SRP implementation in OpenSSL 1.0.1 before 1.0.1i allow remote attackers to cause a denial of service (application crash) or possibly have unspecified other impact via an invalid SRP (1) g, (2) A, or (3) B parameter. | https://git.openssl.org/gitweb/?p=openssl.git;a=commit;h=4a23b12a031860253b58d503f296377ca076427b |
| Openssl | 1.0.1e | CVE-2014-3511 | The ssl23_get_client_hello function in s23_srvr.c in OpenSSL 1.0.1 before 1.0.1i allows man-in-the-middle attackers to force the use of TLS 1.0 by triggering ClientHello message fragmentation in communication between a client and server that both support later TLS versions, related to a "protocol downgrade" issue. | https://git.openssl.org/gitweb/?p=openssl.git;a=commit;h=280b1f1ad12131defcd986676a8fc9717aaa601b |
| Openssl | 1.0.1e | CVE-2014-3513 | Memory leak in d1_srtp.c in the DTLS SRTP extension in OpenSSL 1.0.1 before 1.0.1j allows remote attackers to cause a denial of service (memory consumption) via a crafted handshake message. | https://git.openssl.org/gitweb/?p=openssl.git;a=commit;h=2b0532f3984324ebe1236a63d15893792384328d |
| Openssl | 1.0.1e | CVE-2014-3566 | The SSL protocol 3.0, as used in OpenSSL through 1.0.1i and other products, uses nondeterministic CBC padding, which makes it easier for man-in-the-middle attackers to obtain cleartext data via a padding-oracle attack, aka the "POODLE" issue. | Update Openssl version to 1.0.1j. |
| Openssl | 1.0.1e | CVE-2014-3567 | Memory leak in the tls_decrypt_ticket function in t1_lib.c in OpenSSL before 0.9.8zc, 1.0.0 before 1.0.0o, and 1.0.1 before 1.0.1j allows remote attackers to cause a denial of service (memory consumption) via a crafted session ticket that triggers an integrity-check failure. | https://git.openssl.org/gitweb/?p=openssl.git;a=commit;h=7fd4ce6a997be5f5c9e744ac527725c2850de203 |
| Openssl | 1.0.1e | CVE-2014-3568 | OpenSSL before 0.9.8zc, 1.0.0 before 1.0.0o, and 1.0.1 before 1.0.1j does not properly enforce the no-ssl3 build option, which allows remote attackers to bypass intended access restrictions via an SSL 3.0 handshake, related to s23_clnt.c and s23_srvr.c. | https://git.openssl.org/gitweb/?p=openssl.git;a=commit;h=26a59d9b46574e457870197dffa802871b4c8fc7 |
| Openssl | 1.0.1a | CVE-2014-3513 | Memory leak in d1_srtp.c in the DTLS SRTP extension in OpenSSL 1.0.1 before 1.0.1j allows remote attackers to cause a denial of service (memory consumption) via a crafted handshake message. | https://git.openssl.org/gitweb/?p=openssl.git;a=commit;h=2b0532f3984324ebe1236a63d15893792384328d |
| Openssl | 1.0.1a | CVE-2014-3566 | The SSL protocol 3.0, as used in OpenSSL through 1.0.1i and other products, uses nondeterministic CBC padding, which makes it easier for man-in-the-middle attackers to obtain cleartext data via a padding-oracle attack, aka the "POODLE" issue. | Update Openssl version to 1.0.1j. |
| Openssl | 1.0.1a | CVE-2014-3567 | Memory leak in the tls_decrypt_ticket function in t1_lib.c in OpenSSL before 0.9.8zc, 1.0.0 before 1.0.0o, and 1.0.1 before 1.0.1j allows remote attackers to cause a denial of service (memory consumption) via a crafted session ticket that triggers an integrity-check failure. | https://git.openssl.org/gitweb/?p=openssl.git;a=commit;h=7fd4ce6a997be5f5c9e744ac527725c2850de203 |
| Openssl | 1.0.1a | CVE-2014-3568 | OpenSSL before 0.9.8zc, 1.0.0 before 1.0.0o, and 1.0.1 before 1.0.1j does not properly enforce the no-ssl3 build option, which allows remote attackers to bypass intended access restrictions via an SSL 3.0 handshake, related to s23_clnt.c and s23_srvr.c. | https://git.openssl.org/gitweb/?p=openssl.git;a=commit;h=26a59d9b46574e457870197dffa802871b4c8fc7 |
|  |  | CVE-2014-3568 | OpenSSL before 0.9.8zc, 1.0.0 before 1.0.0o, and 1.0.1 before 1.0.1j does not properly enforce the no-ssl3 build option, which allows remote attackers to bypass intended access restrictions via an SSL 3.0 handshake, related to s23_clnt.c and s23_srvr.c. | https://git.openssl.org/gitweb/?p=openssl.git;a=commit;h=26a59d9b46574e457870197dffa802871b4c8fc7 |
|  |  | CVE-2014-3567 | Memory leak in the tls_decrypt_ticket function in t1_lib.c in OpenSSL before 0.9.8zc, 1.0.0 before 1.0.0o, and 1.0.1 before 1.0.1j allows remote attackers to cause a denial of service (memory consumption) via a crafted session ticket that triggers an integrity-check failure. | https://git.openssl.org/gitweb/?p=openssl.git;a=commit;h=7fd4ce6a997be5f5c9e744ac527725c2850de203 |
|  |  | CVE-2014-3566 | The SSL protocol 3.0, as used in OpenSSL through 1.0.1i and other products, uses nondeterministic CBC padding, which makes it easier for man-in-the-middle attackers to obtain cleartext data via a padding-oracle attack, aka the "POODLE" issue. | Update Openssl version to 1.0.1j. |
|  |  | CVE-2014-3513 | Memory leak in d1_srtp.c in the DTLS SRTP extension in OpenSSL 1.0.1 before 1.0.1j allows remote attackers to cause a denial of service (memory consumption) via a crafted handshake message. | https://git.openssl.org/gitweb/?p=openssl.git;a=commit;h=2b0532f3984324ebe1236a63d15893792384328d |
|  |  | CVE-2014-2851 | Integer overflow in the ping_init_sock function in net/ipv4/ping.c in the Linux kernel through 3.14.1 allows local users to cause a denial of service (use-after-free and system crash) or possibly gain privileges via a crafted application that leverages an improperly managed reference counter. | https://git.kernel.org/cgit/linux/kernel/git/davem/net.git/commit/?id=b04c46190219a4f845e46a459e3102137b7f6cac |
|  |  | CVE-2013-1763 | Array index error in the __sock_diag_rcv_msg function in net/core/sock_diag.c in the Linux kernel before 3.7.10 allows local users to gain privileges via a large family value in a Netlink message. | http://git.kernel.org/cgit/linux/kernel/git/torvalds/linux.git/commit/?id=6e601a53566d84e1ffd25e7b6fe0b6894ffd79c0 |
|  |  | CVE-2014-4943 | The PPPoL2TP feature in net/l2tp/l2tp_ppp.c in the Linux kernel through 3.15.6 allows local users to gain privileges by leveraging data-structure differences between an l2tp socket and an inet socket. | http://git.kernel.org/cgit/linux/kernel/git/torvalds/linux.git/commit/?id=3cf521f7dc87c031617fd47e4b7aa2593c2f3daf |
| Samba | 3.0.37 | CVE-2015-5252 | vfs.c in smbd in Samba 3.x and 4.x before 4.1.22, 4.2.x before 4.2.7, and 4.3.x before 4.3.3, when share names with certain substring relationships exist, allows remote attackers to bypass intended file-access restrictions via a symlink that points outside of a share. | https://git.samba.org/?p=samba.git;a=commit;h=4278ef25f64d5fdbf432ff1534e275416ec9561e |
| linux kernel | 3.4.5 | CVE-2015-1805 | The (1) pipe_read and (2) pipe_write implementations in fs/pipe.c in the Linux kernel before 3.16 do not properly consider the side effects of failed __copy_to_user_inatomic and __copy_from_user_inatomic calls, which allows local users to cause a denial of service (system crash) or possibly gain privileges via a crafted application, aka an "I/O vector array overrun." | http://git.kernel.org/cgit/linux/kernel/git/torvalds/linux.git/commit/?id=637b58c2887e5e57850865839cc75f59184b23d1 |
| Android | 4.4_r1 | CVE-2016-0774 | Back in June of 2015, CVE-2015-1805 a kernel patch was released to implement a fix for vectored pipe read and write functionality which could potentially result in memory corruption. A local, unprivileged user could use the flaw in an unpatched kernel to crash the system or escalate their privileges on the system. Recently it was found that the fix for this issue incorrectly kept buffer offset/length in sync on a failed atomic read. This could result in a pipe buffer state corruption – and a local, unprivileged user could use this to crash the system / leak kernel memory to the user space. | Merge the patches. Refer to https://cve.mitre.org/cgi-bin/cvename.cgi?name=CVE-2016-0774 |
|  |  | CVE-2016-2438 | ** REJECT ** DO NOT USE THIS CANDIDATE NUMBER. ConsultIDs: CVE-2016-2547, CVE-2016-2548. Reason: This candidate is a duplicate of CVE-2016-2547 and CVE-2016-2548. Notes: All CVE users should reference CVE-2016-2547 and/or CVE-2016-2548 instead of this candidate. All references and descriptions in this candidate have been removed to prevent accidental usage. | Merge the Google 2016-4# patch |
| Openssl | 1.0.1a | CVE-2016-2105 | Integer overflow in the EVP_EncodeUpdate function in crypto/evp/encode.c in OpenSSL before 1.0.1t and 1.0.2 before 1.0.2h allows remote attackers to cause a denial of service (heap memory corruption) via a large amount of binary data. | https://git.openssl.org/?p=openssl.git;a=commit;h=5b814481f3573fa9677f3a31ee51322e2a22ee6a |
|  |  | CVE-2016-2106 | Integer overflow in the EVP_EncryptUpdate function in crypto/evp/evp_enc.c in OpenSSL before 1.0.1t and 1.0.2 before 1.0.2h allows remote attackers to cause a denial of service (heap memory corruption) via a large amount of data. | https://git.openssl.org/?p=openssl.git;a=commit;h=3f3582139fbb259a1c3cbb0a25236500a409bf26 |
|  |  | CVE-2016-2107 | The AES-NI implementation in OpenSSL before 1.0.1t and 1.0.2 before 1.0.2h does not consider memory allocation during a certain padding check, which allows remote attackers to obtain sensitive cleartext information via a padding-oracle attack against an AES CBC session, NOTE: this vulnerability exists because of an incorrect fix for CVE-2013-0169. | https://git.openssl.org/?p=openssl.git;a=commit;h=68595c0c2886e7942a14f98c17a55a88afb6c292 |
|  |  | CVE-2016-2108 | The ASN.1 implementation in OpenSSL before 1.0.1o and 1.0.2 before 1.0.2c allows remote attackers to execute arbitrary code or cause a denial of service (buffer underflow and memory corruption) via an ANY field in crafted serialized data, aka the "negative zero" issue. | https://git.openssl.org/?p=openssl.git;a=commit;h=3661bb4e7934668bd99ca777ea8b30eedfafa871 |
|  |  | CVE-2016-2109 | The asn1_d2i_read_bio function in crypto/asn1/a_d2i_fp.c in the ASN.1 BIO implementation in OpenSSL before 1.0.1t and 1.0.2 before 1.0.2h allows remote attackers to cause a denial of service (memory consumption) via a short invalid encoding. | https://git.openssl.org/?p=openssl.git;a=commit;h=c62981390d6cf9e3d612c489b8b77c2913b25807 |
|  |  | CVE-2016-2176 | The X509_NAME_oneline function in crypto/x509/x509_obj.c in OpenSSL before 1.0.1t and 1.0.2 before 1.0.2h allows remote attackers to obtain sensitive information from process stack memory or cause a denial of service (buffer over-read) via crafted EBCDIC ASN.1 data. | https://git.openssl.org/?p=openssl.git;a=commit;h=2919516136a4227d9e6d8f2fe66ef976aaf8c561 |
| Wifi |  | CVE-2016-0801 | The Broadcom Wi-Fi driver in the kernel in Android 4.x before 4.4.4, 5.x before 5.1.1 LMY49G, and 6.x before 2016-02-01 allows remote attackers to execute arbitrary code or cause a denial of service (memory corruption) via crafted wireless control message packets, aka internal bug 25662029. | Merge the *.ko patches from Broadcom. |
|  |  | CVE-2016-0802 | The Broadcom Wi-Fi driver in the kernel in Android 4.x before 4.4.4, 5.x before 5.1.1 LMY49G, and 6.x before 2016-02-01 allows remote attackers to execute arbitrary code or cause a denial of service (memory corruption) via crafted wireless control message packets, aka internal bug 25306181. | Merge the *.ko patches from Broadcom. |
| Openssl |  | CVE-2015-8816 | The hub_activate function in drivers/usb/core/hub.c in the Linux kernel before 4.3.5 does not properly maintain a hub-interface data structure, which allows physically proximate attackers to cause a denial of service (invalid memory access and system crash) or possibly have unspecified other impact by unplugging a USB hub device. | http://git.kernel.org/cgit/linux/kernel/git/torvalds/linux.git/commit/?id=e50293ef9775c5f1cf3fcc093037dd6a8c5684ea |
|  |  | CVE-2016-0723 | Race condition in the tty_ioctl function in drivers/tty/tty_io.c in the Linux kernel through 4.4.1 allows local users to obtain sensitive information from kernel memory or cause a denial of service (use-after-free and system crash) by making a TIOCGETD ioctl call during processing of a TIOCSETD ioctl call | http://git.kernel.org/cgit/linux/kernel/git/torvalds/linux.git/commit/?id=5c17c861a357e9458001f021a7afa7aab9937439 |
|  |  | CVE-2016-3757 | The print_maps function in toolbox/lsof.c in Android 4.x before 4.4.4, 5.0.x before 5.0.2, 5.1.x before 5.1.1, and 6.x before 2016-07-01 allows user-assisted attackers to gain privileges via a crafted application that attempts to list a long name of a memory-mapped file, aka internal bug 28175237. NOTE: print_maps is not related to the Vic Abell lsof product. | http://git.kernel.org/cgit/linux/kernel/git/torvalds/linux.git/commit/?id=5c17c861a357e9458001f021a7afa7aab9937439 |
|  |  | CVE-2016-2842 | The doapr_outch function in crypto/bio/b_print.c in OpenSSL 1.0.1 before 1.0.1s and 1.0.2 before 1.0.2g does not verify that a certain memory allocation succeeds, which allows remote attackers to cause a denial of service (out-of-bounds write or memory consumption) or possibly have unspecified other impact via a long string, as demonstrated by a large amount of ASN.1 data, a different vulnerability than CVE-2016-0799. | https://git.openssl.org/?p=openssl.git;a=commit;h=578b956fe741bf8e84055547b1e83c28dd902c73 |
|  |  | CVE-2015-2686 | net/socket.c in the Linux kernel 3.19 before 3.19.3 does not validate certain range data for (1) sendto and (2) recvfrom system calls, which allows local users to gain privileges by leveraging a subsystem that uses the copy_from_iter function in the iov_iter interface, as demonstrated by the Bluetooth subsystem. | http://git.kernel.org/cgit/linux/kernel/git/torvalds/linux.git/commit/?id=4de930efc23b92ddf88ce91c405ee645fe6e27ea |
|  |  | CVE-2016-3841 | The IPv6 stack in the Linux kernel before 4.3.3 mishandles options data, which allows local users to gain privileges or cause a denial of service (use-after-free and system crash) via a crafted sendmsg system call. | http://git.kernel.org/cgit/linux/kernel/git/torvalds/linux.git/commit/?id=45f6fad84cc305103b28d73482b344d7f5b76f39 |
|  |  | CVE-2016-4482 | The proc_connectinfo function in drivers/usb/core/devio.c in the Linux kernel through 4.6 does not initialize a certain data structure, which allows local users to obtain sensitive information from kernel stack memory via a crafted USBDEVFS_CONNECTINFO ioctl call. | http://git.kernel.org/cgit/linux/kernel/git/torvalds/linux.git/commit/?id=681fef8380eb818c0b845fca5d2ab1dcbab114ee |
| Iptables |  | CVE-2014-9529 | Race condition in the key_gc_unused_keys function in security/keys/gc.c in the Linux kernel through 3.18.2 allows local users to cause a denial of service (memory corruption or panic) or possibly have unspecified other impact via keyctl commands that trigger access to a key structure member during garbage collection of a key. | http://git.kernel.org/?p=linux/kernel/git/torvalds/linux-2.6.git;a=commit;h=a3a8784454692dd72e5d5d34dcdab17b4420e74c |
|  |  | CVE-2015-5364 | The (1) udp_recvmsg and (2) udpv6_recvmsg functions in the Linux kernel before 4.0.6 do not properly consider yielding a processor, which allows remote attackers to cause a denial of service (system hang) via incorrect checksums within a UDP packet flood. | http://git.kernel.org/cgit/linux/kernel/git/torvalds/linux.git/commit/?id=beb39db59d14990e401e235faf66a6b9b31240b0 |
|  |  | CVE-2016-4470 | The key_reject_and_link function in security/keys/key.c in the Linux kernel through 4.6.3 does not ensure that a certain data structure is initialized, which allows local users to cause a denial of service (system crash) via vectors involving a crafted keyctl request2 command. | http://git.kernel.org/cgit/linux/kernel/git/torvalds/linux.git/commit/?id=38327424b40bcebe2de92d07312c89360ac9229a |
|  |  | CVE-2016-4998 | The IPT_SO_SET_REPLACE setsockopt implementation in the netfilter subsystem in the Linux kernel before 4.6 allows local users to cause a denial of service (out-of-bounds read) or possibly obtain sensitive information from kernel heap memory by leveraging in-container root access to provide a crafted offset value that leads to crossing a ruleset blob boundary. | http://git.kernel.org/cgit/linux/kernel/git/torvalds/linux.git/commit/?id=6e94e0cfb0887e4013b3b930fa6ab1fe6bb6ba91 |
|  |  | CVE-2015-2922 | The ndisc_router_discovery function in net/ipv6/ndisc.c in the Neighbor Discovery (ND) protocol implementation in the IPv6 stack in the Linux kernel before 3.19.6 allows remote attackers to reconfigure a hop-limit setting via a small hop_limit value in a Router Advertisement (RA) message. | http://git.kernel.org/cgit/linux/kernel/git/torvalds/linux.git/commit/?id=6fd99094de2b83d1d4c8457f2c83483b2828e75a |
|  |  | CVE-2016-6700 | An elevation of privilege vulnerability in libzipfile could enable a local malicious application to execute arbitrary code within the context of a privileged process. This issue is rated as Critical due to the possibility of a local permanent device compromise, which may require reflashing the operating system to repair the device. | Merge the Google 10# patch |
|  |  | CVE-2016-6828 | An elevation of privilege vulnerability in the kernel networking subsystem could enable a local malicious application to execute arbitrary code within the context of the kernel. This issue is rated as Critical due to the possibility of a local permanent device compromise, which may require reflashing the operating system to repair the device. | https://git.kernel.org/cgit/linux/kernel/git/torvalds/linux.git/commit/include/net/tcp.h?id=bb1fceca22492109be12640d49f5ea5a544c6bb4 |
|  |  | CVE-2016-7910 | An elevation of privilege vulnerability in the kernel file system could enable a local malicious application to execute arbitrary code within the context of the kernel. This issue is rated as Critical due to the possibility of a local permanent device compromise, which may require reflashing the operating system to repair the device. | https://git.kernel.org/cgit/linux/kernel/git/stable/linux-stable.git/commit/?id=77da160530dd1dc94f6ae15a981f24e5f0021e84 |
|  |  | CVE-2016-7911 | An elevation of privilege vulnerability in the kernel file system could enable a local malicious application to execute arbitrary code within the context of the kernel. This issue is rated as Critical due to the possibility of a local permanent device compromise, which may require reflashing the operating system to repair the device. | https://git.kernel.org/cgit/linux/kernel/git/stable/linux-stable.git/commit/?id=8ba8682107ee2ca3347354e018865d8e1967c5f4 |
|  |  | CVE-2015-8964 | An information disclosure vulnerability in kernel components including the human interface device driver, file system, and Teletype driver, could enable a local malicious application to access data outside of its permission levels. This issue is rated as High because it could be used to access sensitive data without explicit user permission. | https://git.kernel.org/cgit/linux/kernel/git/stable/linux-stable.git/commit/?id=dd42bf1197144ede075a9d4793123f7689e164bc |
|  |  | CVE-2016-6753 | An information disclosure vulnerability in kernel components, including the process-grouping subsystem and the networking subsystem, could enable a local malicious application to access data outside of its permission levels. This issue is rated as Moderate because it first requires compromising a privileged process | Merge the Google 10# patch |
| Linux kernel | 3.4.6 | CVE-2016-7042 | The proc_keys_show function in security/keys/proc.c in the Linux kernel through 4.8.2, when the GNU Compiler Collection (gcc) stack protector is enabled, uses an incorrect buffer size for certain timeout data, which allows local users to cause a denial of service (stack memory corruption and panic) by reading the /proc/keys file. | https://bugzilla.redhat.com/attachment.cgi?id=1200212 |
|  |  | CVE-2017-0403 | When perf_group_detach is called on a group leader, it should empty its sibling list. Otherwise, when a sibling is later deallocated, list_del_event() removes the sibling's group_entry from its current list, which could be the now-deallocated group leader's sibling list, leading to a potential use-after-free vulnerability. The fix is designed to deallocate the group_entry on the sibling list properly to prevent the potential use-after-free vulnerability. | Merge the Google 12# patch |
|  |  | CVE-2016-6828 | The tcp_check_send_head function in include/net/tcp.h in the Linux kernel before 4.7.5 does not properly maintain certain SACK state after a failed data copy, which allows local users to cause a denial of service (tcp_xmit_retransmit_queue use-after-free and system crash) via a crafted SACK option. | http://git.kernel.org/cgit/linux/kernel/git/torvalds/linux.git/commit/?id=bb1fceca22492109be12640d49f5ea5a544c6bb4 |
|  |  | CVE-2016-7910 | Use-after-free vulnerability in the disk_seqf_stop function in block/genhd.c in the Linux kernel before 4.7.1 allows local users to gain privileges by leveraging the execution of a certain stop operation even if the corresponding start operation had failed. | http://git.kernel.org/cgit/linux/kernel/git/torvalds/linux.git/commit/?id=77da160530dd1dc94f6ae15a981f24e5f0021e84 |
|  |  | CVE-2016-7911 | Race condition in the get_task_ioprio function in block/ioprio.c in the Linux kernel before 4.6.6 allows local users to gain privileges or cause a denial of service (use-after-free) via a crafted ioprio_get system call. | http://git.kernel.org/cgit/linux/kernel/git/torvalds/linux.git/commit/?id=8ba8682107ee2ca3347354e018865d8e1967c5f4 |
|  |  | CVE-2015-8964 | The tty_set_termios_ldisc function in drivers/tty/tty_ldisc.c in the Linux kernel before 4.5 allows local users to obtain sensitive information from kernel memory by reading a tty data structure. | http://git.kernel.org/cgit/linux/kernel/git/torvalds/linux.git/commit/?id=dd42bf1197144ede075a9d4793123f7689e164bc |
|  |  | CVE-2016-6753 | An information disclosure vulnerability in kernel components, including the process-grouping subsystem and the networking subsystem, in Android before 2016-11-05 could enable a local malicious application to access data outside of its permission levels. This issue is rated as Moderate because it first requires compromising a privileged process. Android ID: A-30149174. |  |
| zlib | 1.2.3 | CVE-2016-6700 | An elevation of privilege vulnerability in libzipfile in Android 4.x before 4.4.4, 5.0.x before 5.0.2, and 5.1.x before 5.1.1 could enable a local malicious application to execute arbitrary code within the context of a privileged process. This issue is rated as Critical due to the possibility of a local permanent device compromise, which may require reflashing the operating system to repair the device. Android ID: A-30916186. | Merge the Google 10# patch |
| openssl | 1.0.1e | CVE-2014-8176 | The dtls1_clear_queues function in ssl/d1_lib.c in OpenSSL before 0.9.8za, 1.0.0 before 1.0.0m, and 1.0.1 before 1.0.1h frees data structures without considering that application data can arrive between a ChangeCipherSpec message and a Finished message, which allows remote DTLS peers to cause a denial of service (memory corruption and application crash) or possibly have unspecified other impact via unexpected application data. | https://github.com/openssl/openssl/commit/470990fee0182566d439ef7e82d1abf18b7085d7 |
|  |  | CVE-2015-0292 | Integer underflow in the EVP_DecodeUpdate function in crypto/evp/encode.c in the base64-decoding implementation in OpenSSL before 0.9.8za, 1.0.0 before 1.0.0m, and 1.0.1 before 1.0.1h allows remote attackers to cause a denial of service (memory corruption) or possibly have unspecified other impact via crafted base64 data that triggers a buffer overflow. | https://git.openssl.org/gitweb/?p=openssl.git;a=commit;h=d0666f289ac013094bbbf547bfbcd616199b7d2d |
| kernel | 3.4.5 | CVE-2012-6689 | The netlink_sendmsg function in net/netlink/af_netlink.c in the Linux kernel before 3.5.5 does not validate the dst_pid field, which allows local users to have an unspecified impact by spoofing Netlink messages. | http://git.kernel.org/cgit/linux/kernel/git/torvalds/linux.git/commit/?id=20e1db19db5d6b9e4e83021595eab0dc8f107bef |
| ffmpeg | 2.6.6 | CVE-2016-6164 | Integer overflow in the mov_build_index function in libavformat/mov.c in FFmpeg before 2.8.8, 3.0.x before 3.0.3 and 3.1.x before 3.1.1 allows remote attackers to have unspecified impact via vectors involving sample size. | http://git.videolan.org/gitweb.cgi/ffmpeg.git/?a=commit;h=8a3221cc67a516dfc1700bdae3566ec52c7ee823 |