MF286_MDM9230_LTE(1_2_3_5_7_8_20_28_38_40_41)_W(1_2_5_8)_StandardNV Release Notes

ZTE® Proprietary

Restricted Distribution: This document contains critical information about ZTE products and may not be distributed to anyone else without the approval from ZTE.

All data and information contained in or disclosed by this document is confidential and proprietary information of ZTE. ZTE is a registered trademark and registered service mark of ZTE Corporation.

ZTE Corporation

Address : ZTE Plaza , Keji Road South , Hi-tech Industrial Park , Nanshan District , Shenzhen , P.R.China , 518057

© 2005 ZTE Corporation. All rights reserved.

目录

1 版本信息	3

2 版本历史	3

3 修改记录	3

# 1 版本信息

表1-1 版本信息

| 基带平台 | MDM9230 |
| --- | --- |
| 射频平台 | WTR3925 |
| 功放 | SKY78071, SKY77807 |
| 主开关 | SKY78071 |
| 分集开关 | QCA1040 |
| 软件版本号 |  |
| 对应的高通软件版本号 | 3012 |

# 2 版本历史

表 2-1版本历史

| 日期 | 说明 |
| --- | --- |
| 2015-07-09 | 初始版本： MF286_MDM9230_LTE(1_2_3_5_7_8_20_28_38_40_41)_W(1_2_5_8)_StandardNV(1.0) |
| 2015-07-30 | MF286_MDM9230_LTE(1_2_3_5_7_8_20_28_38_40_41)_W(1_2_5_8)_StandardNV(1.1) |
| 2015-8-28 | MF286_MDM9230_LTE(1_2_3_5_7_8_20_28_38_40_41)_W(1_2_5_8)_StandardNV(1.2) |
| 2015-9-25 | MF286_MDM9230_LTE(1_2_3_5_7_8_20_28_38_40_41)_W(1_2_5_8)_StandardNV(1.3) |
| 2015-10-10 | MF286_MDM9230_LTE(1_2_3_5_7_8_20_28_38_40_41)_W(1_2_5_8)_StandardNV(1.4) |
| 2016-03-01 | MF286_MDM9230_LTE(1_2_3_5_7_8_20_28_38_40_41)_W(1_2_5_8)_StandardNV(1.5) |
| 2016-05-25 | MF286_MDM9230_LTE(1_2_3_5_7_8_20_28_38_40_41)_W(1_2_5_8)_StandardNV(1.6) |
| 2016-11-16 | MF286_MDM9230_LTE(1_2_3_5_7_8_20_28_38_40_41)_W(1_2_5_8)_StandardNV(1.7) |

# 3 修改记录

| 对应版本：MF286_MDM9230_LTE(1_2_3_5_7_8_20_28_38_40_41)_W(1_2_5_8)_StandardNV.qcn（V1.7） |  |  |  |  |  | 提交人 | 杨永清/郭法永 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| RF验证版本对应高通基线 |  |  |  | 3012 |  |  |  |  |
| 下行CA组合 |  | 异频CA | B1+B8，B3+B7，B3+B8，B3+B20，B3+B28，B7+B20，B7+B28，B7+B8 |  |  |  |  |  |
|  |  | 同频CA | B3+B3，B7+B7，B3C, B7C , B38C，B40C |  |  |  |  |  |
| 上行CA组合 |  | 异频CA | 无 |  |  |  |  |  |
|  |  | 同频CA | 无 |  |  |  |  |  |
| NV ID | 原值 |  |  | 修改值 | 修改原因 |  |  | 波及分析 |
| 27502 | 2050 |  |  | 1650 | 增加EDGE 1800PVT余量 |  |  | 无 |
| 27503 | 2050 |  |  | 1650 | 增加EDGE 1900PVT余量 |  |  | 无 |
| 增加B1+B8 |  |  |  |  | 增加1+8 CA组合，重新校准 |  |  |  |

| 对应版本：MF286_MDM9230_LTE(1_2_3_5_7_8_20_28_38_40_41)_W(1_2_5_8)_StandardNV.qcn（V1.6） |  |  |  |  |  | 提交人 | 郭法永 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| RF验证版本对应高通基线 |  |  |  | 3012 |  |  |  |  |
| 下行CA组合 |  | 异频CA | B3+B7，B3+B8，B3+B20，B3+B28，B7+B20，B7+B28，B7+B8 |  |  |  |  |  |
|  |  | 同频CA | B3+B3，B7+B7，B3C, B7C , B38C，B40C |  |  |  |  |  |
| 上行CA组合 |  | 异频CA | 无 |  |  |  |  |  |
|  |  | 同频CA | 无 |  |  |  |  |  |
| NV ID | 原值 |  |  | 修改值 | 修改原因 |  |  | 波及分析 |
| 25210 | 0x8080044 |  |  | 0x80800c4 | 增加CA B3+B8 |  |  | 无 |
| 25235 |  |  |  | 0x44 | 增加CA B3+B28 |  |  | 无 |
| 针对CA需求，对标准NV进行了重新校准，生成新的射频NV |  |  |  |  |  |  |  |  |

| 对应版本：MF286_MDM9230_LTE(1_2_3_5_7_8_20_28_38_40_41)_W(1_2_5_8)_StandardNV.qcn（V1.5） |  |  |  |  |  | 提交人 | 郭法永 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| RF验证版本对应高通基线 |  |  |  | 3012 |  |  |  |  |
| 下行CA组合 |  | 异频CA | B3+B7，B3+B20，B3+B28，B7+B20，B7+B28，B7+B8 |  |  |  |  |  |
|  |  | 同频CA | B3+B3，B7+B7，B3C, B7C , B38C，B40C |  |  |  |  |  |
| 上行CA组合 |  | 异频CA | 无 |  |  |  |  |  |
|  |  | 同频CA | 无 |  |  |  |  |  |
| NV ID | 原值 |  |  | 修改值 | 修改原因 |  |  | 波及分析 |
| 24938 | 1,-500 |  |  | 1,-440 | 修改B7最小功率标称值，改善非信令测试 |  |  | 无 |

| 对应版本：MF286_MDM9230_LTE(1_2_3_5_7_8_20_28_38_40_41)_W(1_2_5_8)_StandardNV.qcn（V1.4） |  |  |  |  |  | 提交人 | 郭法永 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| RF验证版本对应高通基线 |  |  |  | 3012 |  |  |  |  |
| 下行CA组合 |  | 异频CA | B3+B7，B3+B20，B3+B28，B7+B20，B7+B28，B7+B8 |  |  |  |  |  |
|  |  | 同频CA | B3+B3，B7+B7，B3C, B7C , B38C，B40C |  |  |  |  |  |
| 上行CA组合 |  | 异频CA | 无 |  |  |  |  |  |
|  |  | 同频CA | 无 |  |  |  |  |  |
| NV ID | 原值 |  |  | 修改值 | 修改原因 |  |  | 波及分析 |
| 27472 | 0x33339 |  |  | 0x3333B | HSDPA动态功控问题 |  |  | 无 |

| 对应版本：MF286_MDM9230_LTE(1_2_3_5_7_8_20_28_38_40_41)_W(1_2_5_8)_StandardNV.qcn（V1.3） |  |  |  |  |  | 提交人 | 郭法永 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| RF验证版本对应高通基线 |  |  |  |  |  |  |  |  |
| 下行CA组合 |  | 异频CA |  |  |  |  |  |  |
|  |  | 同频CA |  |  |  |  |  |  |
| 上行CA组合 |  | 异频CA | 无 |  |  |  |  |  |
|  |  | 同频CA | 无 |  |  |  |  |  |
| NV ID | 原值 |  |  | 修改值 | 修改原因 |  |  | 波及分析 |
| 1196,1201,1870,3697,1198,1203,1872,3699 |  |  |  | AGC_PA_ON_RISE_DELAY值修改为2974 AGC_TX_ON_RISE_DELAY值修改为3435 | Ｗ开环功控问题 |  |  | 无 |
| 28148, 28150, 28156, 28158 |  |  |  | TX_ROT_ANGLE NV调整 | W相位不连续性问题 |  |  | 无 |
| 539， 1183， |  |  |  | W2100和W1900功率NV降低至152 | W高频功率偏高 |  |  | 无 |

| NV版本： MF251_MDM9230_LTE(1_2_3_5_7_8_20_28_38_39_40_41)_W(2100_1900_900_850)_TD-SCDMA(34_39_40)_StandardNV（1.2) |  |  |  | 提交人 | 郭法永 |
| --- | --- | --- | --- | --- | --- |
| RF验证对应的高通基线 |  | 3012 |  |  |  |
| NV ID | 原值 | 修改值 | 修改原因 |  | 波及分析 |
| 27354，27355，27358，27359，27431，27432，27433，27435，27437，27438，27448，27454，27459，27461，27462，27466，27468，27469， | 优化前 | 跑FBRX CHAR自动生成 | FBRX CHAR NV项 |  | 无 |
| 26651，26653，26655，26659，26661，26663，26673，26679，26683，26687，26689，26866，26868，26874，26876，27212，27214，27215 | 30，-3，-26 | 30，-7，-27 | FBRX GAIN切换点调整 |  | 无 |
| 6554，20155 | 未激活 | （10，5）；（10，5） | B7，B20的PVT问题 |  | 无 |

| NV版本： MF251_MDM9230_LTE(1_2_3_5_7_8_20_28_38_39_40_41)_W(2100_1900_900_850)_TD-SCDMA(34_39_40)_StandardNV（1.1) |  |  |  | 提交人 | 郭法永 |
| --- | --- | --- | --- | --- | --- |
| RF验证对应的高通基线 |  | 3012 |  |  |  |
| NV ID | 原值 | 修改值 | 修改原因 |  | 波及分析 |
| 28170，28171，28172，28174，28176，28177，28187，28193，28194，28199，28202，28204，28205，28206，28216，28217，28220，28221 |  | NA | FBRX CHAR NV的压缩版本，与生成的FBRX CHAR NV功能相同，但优先使用 |  | 无 |
| 27354，27355，27358，27359，27431，27432，27433，27435，27437，27438，27448，27454，27459，27461，27462，27466，27468，27469， | NA | 跑FBRX CHAR自动生成 | FBRX CHAR NV项 |  | 无 |
| 28148，28150，28156，28158 |  |  | PA静态NV配置，调整切换点及切换点补偿，优化内环 |  | 无 |
| 25076，25077，25078，25079 |  | 最高等级功率压至：高频2900，低频3170，其任各级均降0.5dB | GSM压功率 |  |  |

# 4 备注

| 对应版本 MF286_MDM9230_LTE(1_2_3_5_7_8_20_28_38_40_41)_W(1_2_5_8)_StandardNV(1.0) |  |  | 提交人 |  | 郭法永 |
| --- | --- | --- | --- | --- | --- |
| 备注信息概述： |  |  |  |  |  |
| 第一版NV说明：根据高通3011版本xml文件，更改了GSM、WCDMA和LTE的频段配置信息，使用QDART（4.8.27版本）进行校准后形成射频部分，并且验证了射频指标正常以后，从主板内备份出QCN文件，协议部分参照3011标准NV进行修改，然后再次验证射频工作正常。这样就形成标准NV。 |  |  |  |  |  |
| NV ID | 原值 | 修改值 |  | 修改原因 |  |