ZM8630A_MDM9225_LTE(1_2_3_4_5_7_8_20_28_38_40)_W(2100_1900_AWS_900_850)_StandardNV Release Notes

Dec. 12. 2016

ZTE® Proprietary

Restricted Distribution: This document contains critical information about ZTE products and may not be distributed to anyone else without the approval from ZTE.

All data and information contained in or disclosed by this document is confidential and proprietary information of ZTE. ZTE is a registered trademark and registered service mark of ZTE Corporation.

ZTE Corporation

Address : ZTE Plaza , Keji Road South , Hi-tech Industrial Park , Nanshan District , Shenzhen , P.R.China , 518057

© 2005 ZTE Corporation. All rights reserved.

目录

1 版本信息	3

2 版本历史	3

3 修改记录	3

# 1 版本信息

表1-1 版本信息

| 基带平台 | MDM9225 |
| --- | --- |
| 射频平台 | WTR1625L |
| WCDMA功放 | SKY77629-21 |
| GSM/EDGE功放 | SKY77629-21 |
| 多模开关型号 | SKY13491 |
| 软件版本号 |  |
| 对应的高通软件版本号 | 2031 |

# 2 版本历史

表 2-1版本历史

| 日期 | 说明 |
| --- | --- |
| 2016-12-12 | 初始版本： ZM8630A_MDM9225_LTE(1_2_3_5_7_8_20_38)_W(2100_1900_900_850)_StandardNV(1.0) |

# 3 修改记录

| 对应版本 ZM8630A_MDM9225_LTE(1_2_3_4_5_7_8_20_28_38_40_41)_W(2100_1900_AWS_900_850)_StandardNV |  |  | 提交人 |  | 平鹏飞 |  |
| --- | --- | --- | --- | --- | --- | --- |
| 对应高通分支 |  | 2031 |  |  |  |  |
| 需升级项目及PV/EN版本说明 |  | 所有项目版本均需升级 |  |  |  |  |
| NV ID | 原值 | 修改值 |  | 修改原因 |  | 波及分析 |

# 4 备注

| 对应版本 ZM8630A_MDM9225_LTE(1_2_3_5_7_8_20_38)_W(2100_1900_900_850)_StandardNV(1.0) |  |  | 提交人 |  | 平鹏飞 |
| --- | --- | --- | --- | --- | --- |
| 备注信息概述： |  |  |  |  |  |
| 第一版NV说明：ZM8630A是ZM8630的改版项目，其标准NV是从ZM8630 NV2.6版本升级生成。 |  |  |  |  |  |
| NV ID | 原值 | 修改值 |  | 修改原因 |  |

| 对应版本 ZM8630_MDM9225_LTE(1_2_3_5_7_8_20_38)_W(2100_1900_900_850)_StandardNV (V1.1) |  |  | 提交人 |  | 郭法永 |
| --- | --- | --- | --- | --- | --- |
| 备注信息概述： |  |  |  |  |  |
| V1.1版本NV说明：在初始版本NV基础上，合入针对EDGE功率信道间不平问题重新制作的2G NV部分的小QCN文件及GPRS/EDGE功率回退的小QCN文件，进行2G配置项加入，LTE/UMTS/GSM各频段压功率修改，及LTE的PA_RANGE_MAP修改，并重新校准，得到该版本NV |  |  |  |  |  |
| NV ID | 原值 | 修改值 |  | 修改原因 |  |
| 946 | 0x04C0 | 0x04E8 |  | 增加2G配置 |  |
| 441 | 未激活 | 0x0380 |  |  |  |
| 70847 | PCL0/1：2950 | 2900 |  | GSM压功率 |  |
| 70848 | PCL0/1：2950 | 2900 |  |  |  |
| 70846 | PCL5/6：3250 | 3150 |  |  |  |
| 70845 | PCL5/6：3250 | 3150 |  |  |  |
| 6710 | 23 | 22 |  | LTE压功率 |  |
| 6553 | 23 | 22 |  |  |  |
| 66645 | 23 | 22 |  |  |  |
| 66126 | 23 | 22 |  |  |  |
| 66083 | 23 | 22 |  |  |  |
| 66405 | 23 | 22 |  |  |  |
| 66725 | 23 | 22 |  |  |  |
| 66804 | 23 | 22 |  |  |  |
| 1183 | 162，162，162，162，162，162，162，162 | 152，152，152，152，152，152，152，152 |  | UMTS压功率 |  |
| 1863 | 167，167，167，167，167，167，167，167 | 157，157，157，157，157，157，157，157 |  |  |  |
| 3690 | 167，167，167，167，167，167，167，167 | 157，157，157，157，157，157，157，157 |  |  |  |
| 539 | 162，162，162，162，162，162，162，162 | 152，152，152，152，152，152，152，152 |  |  |  |
| 6717 | 0，1，2，3 | 0，0，3，3 |  | LTE PA_RANGE_MAP修改 |  |
| 6560 | 0，1，2，3 | 0，0，3，3 |  |  |  |
| 66652 | 0，1，2，3 | 0，0，3，3 |  |  |  |
| 66078 | 0，1，2，3 | 0，0，3，3 |  |  |  |
| 66732 | 0，1，2，3 | 0，0，3，3 |  |  |  |
| 66811 | 0，1，2，3 | 0，0，3，3 |  |  |  |
| 66173 | 0，1，2，2 | 0，0，1，1 |  |  |  |
| 65913 | 0，1，2，3 | 0，0，3，3 |  |  |  |
| 66412 | 0，1，2，2 | 0，1，1，1 |  |  |  |

| 对应版本 ZM8630_MDM9225_LTE(1_2_3_5_7_8_20_38)_W(2100_1900_900_850)_StandardNV (V1.2) |  |  | 提交人 |  | 郭法永 |
| --- | --- | --- | --- | --- | --- |
| 备注信息概述： |  |  |  |  |  |
| V1.2版本NV说明：在V1.1版本NV基础上，对LTE欧亚频段的切换点NV及相位不连续性相关NV进行修改，并重新校准，得到该版本NV。 |  |  |  |  |  |
| NV ID | 原值 | 修改值 |  | 修改原因 |  |
| 6719 | 860,830,860,830,765,735 | 860,830,860,830,860,830 |  | LTE切换点 |  |
| 6562 | 82,3,52,3, 82,3,52,3, 2,3,-28,2 | 82,3,52,3, 82,3,52,3, 82,3,52,3 |  |  |  |
| 66654 | 860,830,860,830,765,735 | 860,830,860,830,860,830 |  |  |  |
| 66174 | 860,830,860,830,765,735 | 860,830,0,0,0,0 |  |  |  |
| 66077 | 860,830,860,830,765,735 | 860,830,860,830,860,830 |  |  |  |
| 66414 | 865,835,770,740,0,0 | 860,830,0,0,0,0 |  |  |  |
| 66734 | 860,830,860,830,765,735 | 860,830,860,830,860,830 |  |  |  |
| 66813 | 未激活 | 860,830,860,830,860,830 |  |  |  |
| 1795 | 0 | 300 |  | UMTS相位不连续性相关 |  |
| 2046 | 0 | 380 |  |  |  |
| 2071 | 0 | 480 |  |  |  |
| 3722 | 0 | 400 |  |  |  |

| 对应版本 ZM8630_MDM9225_LTE(1_2_3_4_5_7_8_20_28_38)_W(2100_1900_900_850)_StandardNV (V1.9) |  |  | 提交人 |  | 董金涛 |
| --- | --- | --- | --- | --- | --- |
| 备注信息概述： |  |  |  |  |  |
| V1.9版本NV说明：在V1.8版本NV基础上，对LTE频段新增了B4与B28的支持，修改B4,B28最大功率、切换点、PA range map |  |  |  |  |  |
| NV ID | 原值 | 修改值 |  | 修改原因 |  |
| 65906 | 23 | 22 |  | LTE压功率 |  |
| 71221 | 23 | 22 |  |  |  |
| 65913 | 0,1,2,3 | 0,3,3,3 |  | LTE PA_RANGE_MAP修改 |  |
| 71228 | 0,2,2,3 | 0,1,1,1 |  |  |  |
| 65915 | 860,830,860,830,765,735 | 780,750,780,750,780,750 |  | LTE切换点 |  |
| 71230 | 870,840,830,800,730,700 | 780,750,0,0,0,0 |  |  |  |

| 对应版本 ZM8630_MDM9225_LTE(1_2_3_4_5_7_8_20_28_38)_W(2100_1900_1700_900_850)_StandardNV (V1.10) |  |  | 提交人 |  | 董金涛 |
| --- | --- | --- | --- | --- | --- |
| 备注信息概述： |  |  |  |  |  |
| V1.10版本NV说明：在V1.9版本NV基础上，对UMTS新增了BAND4的支持，修改UMTS B4,最大功率、相位不连续性、功控相关NV，并重新校准 |  |  |  |  |  |
| NV ID | 原值 | 修改值 |  | 修改原因 |  |
| 04035 | 152，152，152，152，152，152，152，152 | 157，157，157，157，157，157，157，157 |  | UMTS压功率 |  |
| 04067 | 0 | 400 |  | UMTS相位不连续性相关 |  |
| 04081 | 0 | -2 |  | UMTS内环功控相关 |  |
| 04082 | 0 | 3 |  |  |  |